## Report on Data Recovery for Mission # ___1932-03___________

REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 04-Oct-2017 | See also the technical report here: http://cat.fsl-bsf.scitech.gc.ca/record=b4058614~S1. R.H. |
| 20-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | Yes |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run REORDER to fix channel name used twice | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix channel names, formats and units, as required | Yes |
| Produce track plot to check for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

There were “9” flags – some should be “5” because data were replaced with pad values (comments about them in header); others should be 0 since there never was a value – presume it was not sampled.

Big problem is that the DO flag channel was mistakenly named as the pH flag channel so that there are 2 channels with the same name. REORDER was used to fix this, followed by HEADEDIT. This was complicated by having a variety of names in different casts, so it took many iterations to get everything right.

.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

Tests profile and T-S plots were examined on-screen and no obvious bad data were found.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.