## Report on Data Recovery for Mission # ______1932-04________

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 04-Oct-2017 | See also the technical report here: http://cat.fsl-bsf.scitech.gc.ca/record=b4058614~S1. R.H. |
| 20-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | Yes |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix channel names, formats and units, as required | Yes |
| Produce track plot to check for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check | Yes |
| Test plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Tests profile and T-S plots were examined on-screen. One outlier was noted but had already been flagged.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.