## Report on Data Recovery for Mission # __1938-10____________

REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 22-Aug-2014 | File 1938-10-0015.bot: The start year was changed from 1935 to 1938. |
| 21-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot to check for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check | Yes |
| Test plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

There were “9” flags for some data – in each case the value had been replaced with a pad value, so a “5” flag would be appropriate. There was no “9” in the scale originally used for these data.

The flag definitions were missing from the header so were added in the HEADEDIT text file.

The times are stated to be in UTC but are actually in local time, so ADD TIME CHANNEL was used to add 8 hours and GMT to the headers. This is consistent with other cruises in 1938.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

Tests profile and T-S plots were examined on-screen. The salinity value at 10m for cast #27 looks out of line. The value is 32.24. The original cruise records show indicate that the chlorinity was 17.294 which is equivalent to 31.24. That number looks much better in the plots. The chlorinity conversion was tested on a few other values and no discrepancy was found. Flag 5 was assigned and a note was added to the header.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.