## Report on Data Recovery for Mission #  1950-38_

REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 6-Sept-2024 | Removed empty channels from files 9, 15, 18, 23 |
| 21-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

Flags were adjusted – low salinity surface values were changed from 3 to 1 as they look ok. Flags were changed from 9 to 0 for instances where pad values were entered with no explanation since 9 is not one of the flags used in the MEDS files. A few other flags were changed as the wrong one seemed to have been reversed. Flags were added to obviously bad data such as cast #9 temperatures which were all entered as -72.8; the original data could not be found.

There are DO values in volume and mass units.

No problems were found in the track plot, standards check and cross-reference listing. The header check turned up a few other problems which were fixed.

Test plots were run and no further problems were found.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.