## Report on Data Recovery for Mission # __1950-44___

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 21-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

There were many salinity reversals between 0 and 1m – usually the 1m sample was flagged – this is arbitrary – often that one seems more believable than the surface one. Occasionally the flags were changed, sometimes left in the absence of information, but it is quite possible that surface slicks or debris affect the 0m sample. It is assumed that users will not expect great accuracy near the surface and the differences are not usually very large given the local gradients are high.

No problems were found in the track plot, cross-reference listing, and standards check. A faulty event # and a case of depth in Feet were found in the header check so those were fixed and the check rerun.

Test plots were run and in some cases showed that values flagged 3 were fine, so those were changed to 1. The profile of temperature looks odd alone, but not in T-S space.

Some DO values were flagged 3 due to very small reversals, but those do occur, and even if there is an analysis problem it is not clear which values are out of line. So the 3 flags were changed to 1. One bottle salinity outlier was found and flagged 3.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.