## Report on Data Recovery for Mission # ___1950-48___________

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 21-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

Data taken from ship at anchor at 4 sites – Casts 1-26 and #78-103 were in the Fraser River – the first set in the North Arm and the last set in the South Arm near Steveston. Casts 27-51 were just offshore near UBC and #52-77 were just offshore of Steveston. There are only salinity data plus depths that appear to be in yards but are said to be in metres. Given the accuracy is limited, this was left unchanged. Near-surface salinity values of 0 were sometimes flagged 1 and sometimes 3. Since there was no change of position, and the data are supported by original deck sheets, there seems no reason for the 3 flags so they were changed to 0. Some 5m values were flagged 3 presumably because of a change from one cast to the next, but this happens fairly regularly and given tidal variations these flags are not justified; they were removed.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.