## Report on Data Recovery for Mission # ___1950-78____

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 21-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | Yes |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

As for cruise 1950-67 the Dissolved Oxygen data look low with a maximum of 5.2mL/L, but they do agree with the deck sheets. The MEDS file entries equal deck sheet values times 0.7 to go from mg/L to mL/L. There could have been an error and the deck sheets are really in mL/L but there is no evidence for that and given the area where these data are found, it is possible to get strong mixing. The units mg/L were in wide use at that time.

The surface saturation varies from 90% to 110% with most values at the low end of that range. Bill Crawford confirms that they are a little low within his data base. But if the values in the original file are assumed to be mL/L, the saturations are too high, 95-150% with most >130%.

The locations may account for low saturations as the lowest values come from an east-west line south of the area from which we usually get data. The two offshore casts to the north have values ~6mL/L and just south of the mouth of Juan de Fuca Strait the values range from 6 to 7mL/L which is reasonable. There seems no justification to change the values.

Flags were removed from a few values that look ok.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and a few outliers were flagged. A deep near-zero salinity value was replaced with a pad value.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.