## Report on Data Recovery for Mission # ____1951-57__________

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 22-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels - time | Yes |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags - 9 to 0, removed few 2 and 3 flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

DO surface saturation calculated based on MEDS values have values vary from about 90 to 100%, with most <95%, but given the region near Haro Strait, these are reasonable results.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and a few values were outside the current climatology but too close to shore for that to be significant, so a few flags based on climatology were removed.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.