## Report on Data Recovery for Mission # _________1951-63_____

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 22-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels – some casts had DO removed | Yes |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

The header check shows DO values lower than expected. As found for some other cruises there are two columns with DO values in the original sheets with labels Titer and Mg/l. But the values look as though the two columns should be labeled mg/L and mL/L; the second column is 0.753 times and 0.753 is the factor recorded on the sheet. The MEDS file has used the second column and multiplied those values by 0.7. There could have been an error and the deck sheets are really in mL/L but there is no evidence for that and given the area where these data are found, it is possible to get strong mixing. No change is justified.

For some casts there are DO values listed without a flag channel; they are clearly wrong and missing from the original cruise records. The DO channel was removed from casts #10, 12, 15, 17, 20, 22, 25, 27, 34, 36, 39, 41, 53.

Flags were fixed in a few casts, mostly 9 changed to 0 for pad values.

No problems were found in the track plot, cross-reference listing and standards check.

Test plots were run and all data are within the current climatology. T-S plots look ok with only a few slightly unstable features in areas where active mixing is likely.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.