## Report on Data Recovery for Mission # __1952-68____________

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 22-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Yes |
| Prepare header comment with details on data source. | Yes |
| Rename files with standard cruise # | Yes |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Yes |
| Fix faulty flags | Yes |
| Run CLEAN to fix header channel limits, pad values | Yes |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Yes |
| Produce track plot and standards check – look for errors | Yes |
| Produce x-ref list – include in documentation folder | Yes |
| Produce Header Check– include in documentation folder | Yes |
| Test Plots | Yes |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

Some flags needed changing 9 to 0 mostly, few flagged values look ok, so flag removed.

The DO values look low. Surface saturation ranged from 65% to 75% with just one value higher than that at 83%. But these are from Juan de Fuca, mostly in the eastern part where mixing is high. The temperature profiles show most casts are well mixed, so low saturations are believable.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and temperatures were low compared to the climatology in the Haro Strait area – those casts look very well mixed and the climatology is too severe for such areas. A few salinity values were low near the bottom, a few near Haro Strait and a few near the mouth of Juan de Fuca. None of these excursions looks indicative of instrumental or analysis errors.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.