Report on Data Recovery for Mission # __1954-21____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | n/a |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments: These data were transcribed by Bernard Minkley in 2009 from a publication of the Fisheries Board of Canada in the library at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving.

**There were quality flags but they were all set to 1 even for pad values. There was no definition of the flag. The flags serve no purpose so the flag channels were removed.

.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and while there are a few minor unstable values, they do not look significant.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.