## Report on Data Recovery for Mission # __1955-94____

 REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 28-Aug-2013 | The MEDS Data Quality Flags were confusing and unreliable since automated quality control methods were used with no consideration of the local conditions. So it was decided to remove them from the files. J.L. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels | n/a |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | Done |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments: These data were collected by Bernard Minkley in 2009 and checked against cruise data records on file at the Institute of Ocean Sciences.

The files were subsequently put through the steps shown above to prepare them for archiving. Most flags were 1; the few that were flagged 3 and 4 did not look like significant outliers – for example, very slight reversals in heavily mixed casts and flagging one value as an outlier when the one above or below might be the ones out of line; none were significantly out of line. 9 flags were changed to 0 to correspond to the MEDS definitions.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and no significant outliers were found. Some temperature values were below the historic minimum of the climatology in use in 2013, but the 3-standard deviation climatology is considered too severe for near-shore casts and there is believed to have been warming since the 1950s. Almost all salinity was within the climatology. T-S plots look reasonable.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.