Report on Data Recovery for Mission # __1957-09____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done earlier |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

This was a complex case with some data deriving from MEDS but with truncated values and some transcribed from data in the publication:

Canadian Joint Committee on Oceanography

Physical, Chemical and Plankton Data Record

Coastal Surveys April 25 to December 17, 1957

Pacific Oceanographic Group, 1958

The truncated MEDS data were restored by comparing with the report.

Phosphate data is included in some files, but not others. In some cases there are pad values which can be replaced with the correct data, but in other cases the channel had never been added.

Temperature and flag channels were given different names in the two types of files, complicating removal of some and standardizing of others.

Flags had been added to the data, but none are found in the original analysis logs or the published data, and almost all flags were set to 1, so they serve no purpose. Those set to 2 or 3 look inappropriate. The flags were removed. Time of observation was included in a few files and that was also removed.

A few interpolated values had been replaced with pad values and those were left in place.

No problems were found in the header checks and standards check. The track plot and cross-reference listing look fine.

Test plots were run and all data fell within the climatology in use in 2013. Most data were stable in T-S space except the bucket samples at the surface were often out of line.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.