Report on Data Recovery for Mission # __1957-14____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done earlier |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

Some of these data derive from MEDS but with truncated values and some missing data that were transcribed from data in the publication:

Canadian Joint Committee on Oceanography

Physical, Chemical and Plankton Data Record

Coastal Surveys April 25 to December 17, 1957

Pacific Oceanographic Group, 1958

The truncated MEDS data were restored by comparing with the report.

The Phosphate channel was included even when there were only pad values.

Flags had been added to the data, but none are found in the original analysis logs or the published data, and almost all flags were set to 1, so they serve no purpose. Those set to 2 or 3 look inappropriate with one possible exception. The flags were removed.

A few interpolated values had been replaced with pad values and those were left in place.

No problems were found in the header checks and standards check. The track plot and cross-reference listing look fine.

Test plots were run and all data fell within the climatology in use in 2013 except for one salinity value (cast #41, 800m) which looks ok in T-S space and is in an area where interleaving is quite possible. Other casts have similar but weaker features. All data were stable in T-S space except the bucket samples at the surface were often out of line.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.