Report on Data Recovery for Mission # __1955-16____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments: Some data for this cruise came from MEDS and some were transcribed by B. Minkley from the publication:

Canadian Joint Committee on Oceanography

Physical, Chemical and Plankton Data Record

Coastal Surveys April 25 to December 17, 1957

Pacific Oceanographic Group, 1958

**There were quality flags but they were mostly set to 1 even for pad values. There were no definitions of the flags though they follow the style of MEDS flags. There are no flags in the original deck sheets and analysis logs or in the publication above. Flag 3 was assigned to many points just below the surface, but it is far more likely that the surface bucket sample is the one that is out of line, but none of those were flagged. The flag channels do not appear to be useful and were removed. A few interpolated values had been replaced with pad values and those were left in place.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and all T and S fell within local climatology except for some low temperature values around 150m that are below the minimum; these look consistent among nearby casts, so likely reflect real conditions. The 3-standard deviation climatology used is too severe for near-shore casts. Cast #43 had an odd, but stable, temperature reversal, but similar features are seen at 2 nearby casts, so it is assumed to be real. Cast #4 has small reversals in T and S that are slightly unstable, but not within the precision of these variables. The values were left in place.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.