Report on Data Recovery for Mission # __1957-20____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** needed 2 runs | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments: The data for this cruise came from the World Ocean Data Base but additional data were added based on the publication:

Canadian Joint Committee on Oceanography

Physical, Chemical and Plankton Data Record

North Pacific Survey July 23 to August 30, 1957

Pacific Oceanographic Group, 1957

**There were quality flags but they were mostly set to 1 even for pad values. There were no definitions of the flags though they follow the style of MEDS flags. There were a few flags around 10m but it is likely the surface bucket samples that were out of line and those were not flagged. The flagged salinity value for cast #37 proved to be a bad entry. The correct value was found in the original analysis records. The flag channels do not appear to be useful and were removed. A few interpolated values had been replaced with pad values and those were left in place.

There were two channels called Flag:Silicate, so REMOVE had to be run twice.

Nitrite data were added.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run and while there are a few minor unstable values at the surface, they do not look significant. All T and S fell within local climatology except for one slight excursion in salinity that looks fine in T-S space.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.