Report on Data Recovery for Mission # __1958-30____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from MEDS by B. Minkley. Disoslved Oxygen was recalibrated and some reformatting was done.

The files were subsequently put through the steps shown above to prepare them for archiving.

**There were quality flags but they were mostly inappropriate, for example, a flag 3 was applied to the first bottle below the surface, usually 2m, for salinity if it was lower than the surface salinity. But the surface samples were bucket samples and more likely to be the ones in error. The only flags that look appropriate were for casts #7 and 91.

No problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run, but since many are in inlets, areas of tidal mixing and rapid change, interpretation is limited. The local climatology is not very useful for all except the offshore casts, especially since the climatology contains data from Jan 1986 to Feb 2008, not 1958. Some casts look unstable but temperature varies by only 0.02Cº in 170m. There was 1 salinity value that did look significantly out of line from cast #91 at 700m. This was offshore and was replaced with a pad value. The original value was entered in a comment in the headers.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.