## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 12-Feb-2006 | Added Loop data from the publication referenced below. |

Cruise 6099 Nutrient data processing notes from Melanie Quennevile:

Cross-referenced Chemistry’s files with files in OSAP archives. The chemistry oxygen data are in different units, the chemistry files have pad values for missing data, and the header information was different.

Chemistry files LPC60001 to 60004 are the same as OSAP 59990094 to 0097.  Chemistry files LPC60005 to 60010 are the same as OSAP 60990001 to 60990006.  Chemistry files LPC60011 to 60018 are the same as OSAP files 59990098 to 59990105. Chemistry files LPC60019 to 60129 are the same as OSAP files 60990007 to 60990117.  Other than these numbering differences, the data were identical.

June 5, 2001:

Imported nutrient data to excel file, removed notes. Put date and time in one cell. Changed event numbers. Converted to IOSSHELL format. Ran X-ref, compared with OSAP archive files:

3 unique files (deep casts) with T, sal, Si, PO4. Files are 9087, 9093, and 9095. Oxygen is in mg-At/L

Silicate data most casts, PO4 some casts

Changed file names of nutrient files to match OSAP archive files. Merged the nutrient data to the OSAP data. Added header information from Chemistry files to the merged files.

For the ‘new’ casts, entered the rest of the T/S data from the data report, converted, added header information.

Saved files in \finished\1960.

Cast 9093: From data report, ‘new’ cast data: Silicate data are suspect (low).

The oxygen (mass) format is OK for .oxn , .oxy files.

Sept 6, 2001:

Converted and added oxygen in umol/kg to casts 6, 7, 8, 9, 48, 53, 56, 57, 58, 59 , 60, 61, 70, 71, 72, 73, 74, 75, 76, 77, 79, 89, 82, 83, 85, 86, 93, 95, 98, 103, 104, 105, 105, 107, 109, 114. Designated .oxy.

# February 4, 2002 – Joe Linguanti

Started work from Melanie’s directory \\OSAPCOMMON\STPCHEM\FINISHED\1960.

- Split the *.OXN, *.OXY and *.HDD files into Line P cruises 6001, 6002, 6003, 6004 and North Pacific cruises 6005, 6006 by renaming them.

# Cruise 1960-01: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 98, Data Record Ocean Weather Station "P" (50:00'N, 145:00'W). December 9, 1959 – January 19, 1961.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

# Cruise 1960-02: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 98, Data Record Ocean Weather Station "P" (50:00'N, 145:00'W). December 9, 1959 – January 19, 1961.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

# Cruise 1960-03: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 98, Data Record Ocean Weather Station "P" (50:00'N, 145:00'W). December 9, 1959 – January 19, 1961.

- Cast 10 was not in the FINISHED directory so file 60990081.hdr was copied to 6003 and renamed to 60030010.new.

- File 60999087 was merged with cast 13 as it is in the data report. Phosphate data was added to file.

- In Sus’s database cast 17 was a composite of 17, 20 and 22. The data from 20 and 22 were removed from cast 17. File 60999093.hdr was renamed to 60030020 and file 60999095 was renamed to 60030022 and oxygen units were converted from mg.At/L to mL/L.

- The nutrient data for cast 26 was not in the file in the FINISHED directory so file 60990098.IOS was copied from the PROCESSING directory and the nutrient data was merged to cast 26.

- HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

# Cruise 1960-04: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 98, Data Record Ocean Weather Station "P" (50:00'N, 145:00'W). December 9, 1959 – January 19, 1961.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

## April 2, 2002 – Joe Linguanti

Reference:

McAllister, C.D., MS, 1962. Data Record. Photosynthesis and Chlorophyll a Measurements at Ocean Weather Station "P", July 1959 to November 1961. Fisheries Research Board of Canada, Manuscript Report Series (Oceanographic and Limnological), No. 126, 14 pp.

Two productivity casts (9000 series) containing primary production and chlorophyll data were obtained from the manuscript report series cited in reference.

# Cruise 1960-05: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 63, Oceanographic Data Record North Pacific Survey January 12 to February 10, 1960. IOS Library: FRB-MR-OL-63.

- Files 60990001 to 60990014 from Sus’s database were obtained from North Pacific cruise NP-60-01 pages 13-18 and 119-127. The files were renamed to cruise 60-05.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

# Cruise 1960-06: February 4, 2002 – Joe Linguanti

Reference:

Fisheries Research Board of Canada Manuscript Report Series, (Oceanographic and Limnological), No. 82, Oceanographic Data Record North Pacific Survey July 10 to September 6, 1960. IOS Library: FRB-MR-OL-82.

- Files 60990062 to 60990069, 60990079,60990080, 60990082, 60990083 and 60990085 from Sus’s database were obtained from North Pacific cruise NP-60-03 pages 93-114. The files were renamed to cruise 60-06.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.