Report on Data Recovery for Mission # __1960-42 changed to 1960-05____

## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 6-Sept-2024 | Edited Oxygen:Dissolved in mL/L by removing spurious initial digit; rederived Oxygen:Dissolved in umol/kg. |

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Overlap with archive |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Calculate DO Mass units | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from the World Ocean Data Base by B. Minkley. There was also a published report available and original records. Dissolved Oxygen was recalibrated and some reformatting was done.

Some of these files were found in the archive but most were missing and the archived files had errors in some event numbers. There were 3 sig. figures in salinity in the archive, so the 3rd decimal place was added to the EDT files based on the publication.

The files were subsequently put through the steps shown above to prepare them for archiving. There were many versions of most channel names that required many attempts at editing headers to catch all of them.

**Quality flags had been added but they were inappropriate, mostly the values from shallowest Niskin were flagged whereas the value out of line is far more likely to be the  surface samples which were bucket samples and less accurate. None were found to be appropriate and they were not assigned by the analyst or in the report, so flag channels were removed.

Header checks turned up some errors in time which were checked against bridge logs, and there were some typos that were found from test plots. A few bad values were replaced with pad values. Many format problems were found and corrected.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

There were few excursions from the climatology and none looked significant once the errors were corrected. T-S plots look stable except near the surface and close to shore in well-mixed casts.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.