Report on Data Recovery for Mission # __1960-52 changed to 1960-06____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Overlap with archive |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Calculate DO Mass units | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from the World Ocean Data Base by B. Minkley. There was also a published report available and original records were available. Dissolved Oxygen was recalibrated and some reformatting was done.

Original data can be found in the publication:

Fisheries Research Board of Canada

Manuscript Report Series, (Oceanographic and Limnological), No. 82

Oceanographic Data Record, North Pacific Surveys

July 10 to September 6, 1960.

IOS Library: FRB-MR-OL-82.

The files from 98 to 107 are misnamed and should be 99 to 108. The data for cast #98 was missed. So the files were renamed and a new file was created for 98 based on published values. The station names are wrong in the files and since they are the same as the event numbers, there is no point in having this entry. That entry will be removed in the Head Edit phase.

Some of these files were found in the archive but most were missing. There were 3 sig. figures in salinity in the archive, so the data from the files were exported to a spreadsheet so the salinity could easily be restored to 3 decimal places. The spreadsheet was converted to DAT files and merged with the HED files.

**Quality flags had been added but they were all set to 1, so they were removed.  There was a code in the publication that relates to the precision in the temperature data, but there is no simple relationship to current quality flag channel since the lack of a duplicate was treated in the same was as bad replicates. Those codes were not in the files retrieved from the WODB.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

Profile plots turned up 2 typos; most other excursions from the climatology were small and do not look significant. The only one that stands out is cast #66 which is quite different from nearby casts, with warmer, fresher waters below about 100m, which looks like a Haida Eddy. These data are likely ok. T-S plots turned up one typo; after fixing that all look stable except near the surface and close to shore in well-mixed casts.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.