Report on Data Recovery for Mission # __1960-48____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done – not found |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from MEDS by B. Minkley and include Dissolved Oxygen data which were recovered from the following publication in the IOS library:

Fisheries Research Board of Canada

Manuscript Report Series, (Oceanographic and Limnological), No. 76

Oceanographic Data Record, Coastal Project

June 6 to 16, 1960

IOS Library: FRB-MR-OL-76.

The files were subsequently put through the steps shown above to prepare them for archiving.

**Quality flags had been added but most were found inappropriate, with most on the 5m samples. The surface samples were bucket samples and of lower precision than those at 5m, so it is more likely the surface samples that are out of line which were not flagged. There were 2 salinity values that did look out of line compared to others in a time-series. Those 2 values were replaced with pad values.

One file contained no data - #108 – data were found in the publication, so were inserted in the file.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run. The local climatology contains data from Jan 1986 to Feb 2008, not 1960. All temperature data and most salinity data fell within the 3-standard deviation plots. However, there were some low salinity values in the top 30m in the southern part of the cruise. There was also 1 deeper salinity value that was out of line, and this was at a site with many observations. The value was replaced with a pad value. The only unstable features were at the surface where bucket samples are often out of line, but that is expected given the less precise nature of those measurements.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.