Report on Data Recovery for Mission # __1960-53____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done – not found |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** and Time | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | Done |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from MEDS by B. Minkley. There were no Dissolved Oxygen data.

The data can be found in a publication in the IOS library:

Fisheries Research Board of Canada

Manuscript Report Series, (Oceanographic and Limnological), No. 83

Oceanographic Data Record, Coastal and Seaways Projects

October 3 to 26, 1960.

IOS Library: FRB-MR-OL-83.

The files were subsequently put through the steps shown above to prepare them for archiving.

**Quality flags had been added but all were found inappropriate and they were not assigned by the analyst or in the report, so flag channels were removed.

The salinity data were edited to include 3 decimal places as given in the published report.

No further problems were found in the track plot, cross-reference listing, header checks and standards check.

Test plots were run. All data fell within the local climatology (this is based on data from Jan 1986 to Feb 2008, not 1959). The only unstable features in T-S space were a few surface bucket samples which is expected given the less precise nature of those measurements.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.