Report on Data Recovery for Mission # __1961-65 renamed as 1961-08____

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done –ok |
| Prepare header comment with details on data source. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels** | Done |
| Export data to CSV file and check for faulty flags | Done |
| Fix faulty flags | n/a |
| Calc DO in mass units | Done |
| HeadEdit to add 3rd decimal to sal – edit sal | Done |
| Run CLEAN to fix header channel limits, pad values | Done |
| Run REORDER to get the 2 DO channels together | n/a |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required | Done earlier |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

These files were obtained from World Ocean Data Base and the published report:

Fisheries Research Board of Canada

Manuscript Report Series, (Oceanographic and Limnological), No.101

Oceanographic Data Record, North Pacific Surveys

May 16 to July 1, 1961

IOS Library: FRB-MR-OL-101

Flags had been added by but all were 1 and were not in the original data, so they were removed.

Dissolved oxygen was derived in umol/kg.

Salinity was edited to add 3rd decimal place.

No problems were found in the cross-reference listing, header checks and standards check.

Test plots were run and there were cases of salinity below the local climatology but these were from near-shore casts, so this is not unusual. Moreover the climatology is from another era.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.

Some of these casts were already in the IOS Data Archive named as 1961-08, but there were errors in station names and mission number.