## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 4-Mar-2006 | Added loop data from data reports cited below. |

Cruise 6503: Comparison between chemistry and OSAP files; nutrient data additions by Melanie Quenneville:

Chem files LPC65020 to LPC65053 correspond to OSAP files 65010001 to 65010034.

Not all stations have oxygen values.  Chem files have pad values for the missing data. Oxygen is in different units in chem files.

No nitrate data this cruise.

Otherwise chem data are identical to archive

September 11, 2001:

Changed Oxygen (mL/L) to Oxygen:dissolved:volume and added Oxygen:dissolved:mass (umol/kg) channel.

# February 8, 2002 – Joe Linguanti

Started work from Melanie’s directory \\OSAPCOMMON\STPCHEM\FINISHED\1965\6503.

# Cruise 1965-03: February 8, 2002 – Joe Linguanti

Reference:

Canadian Oceanographic Data Centre. MS, 1966. Ocean Weather Station P",

North Pacific Ocean. 1966 Data Record Series, No. 3 (Apr 17 – Jun 03, 1965).

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

## April 3, 2002 – Joe Linguanti

Reference:

Stephens, K., MS, 1966. Data Record. Primary Production Data from the N.E. Pacific Ocean, January 1964 to December 1965. Fisheries Research Board of Canada, Manuscript Report Series (Oceanographic and Limnological), No. 209.

Three productivity casts (9000 series) containing chlorophyll and primary production data were obtained from John Page’s files.