Report on Data Recovery for Mission # __1965-07__

| Processing Steps | Done? |
| --- | --- |
| Check if these data are in the MasterCruiseList. If it is not obvious from the list look at the archived data to ensure there is no overlap. | Done –no overlap |
| Prepare header comment with details on data source, if missing. | Done |
| Rename files with standard cruise # | Done |
| Remove unwanted channels – flag channels | Done |
| Export data to CSV file and check for faulty flags | All =1 |
| Fix faulty flags | n/a |
| Derive DO in mass units | n/a |
| Run HEADEDIT to add comments, add data description, fix mission name, fix formats and units, as required – add 3rd decimal to SAL | Done |
| Edit salinity to add 3rd decimal place, CLEAN to fix header limits | Done |
| Produce track plot and standards check – look for errors | Done |
| Produce x-ref list – include in documentation folder | Done |
| Produce Header Check– include in documentation folder | Done |
| Test Plots | Done |

Comments:

Files were created in 2010 that were based on original cruise documents said to be stored in the IOS vault, but those documents could not be found in 2013. Time zones and header information could not be checked.

Quality channels had been added in 2010. The only flag that was not =1 was a temperature value that looked out of line. Since the salinity also looks out of line and dissolved oxygen could be out of line, it is likely that the depth is wrong. It was decided to remove that record – a note was made in the header, that the depth was questionable so the record was removed.

REMOVE was run to remove quality flag channels

No problems were found in the header check, cross-reference listing and standards check.

Test plots were run. The available climatology was based on averages over large areas and mostly suits open ocean data.  So while most data fell within those limits, there are outliers especially in narrow inlets, and those are not considered signs of poor data quality. T-S plots were mostly stable with only minor exceptions.

A cross-reference listing and a header check report may be found in the DOC folder in the OSD_DATA_ARCHIVE.