## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 19-Mar-2006 | Added Loop data from the publications referenced below. |

Cruise 6902 by Melanie Quenneville.

Comparison between chemistry and OSAP archive files:

Not all stations have oxygen/nutrient values; see record sheet. Chem files have pad values for the missing data.Oxygen is in different units in chem files.

CTD or STD files not distinguished from bottle files in chem's files.

No Phosphate or Silicate data this cruise; chem has pad values.

Cast #1 contains NO3 data, not on archive. Chlorophyll and productivity data were also collected.

Some cruise dates overlap. Chem files do not distinguish between cruises, therefore note offset in cast numbers.

Header information: : Added chemistry header information to OSAP files.

Otherwise data are identical to archive. Note offset in cast numbers.

Aug 30: The header information for cast 9001 was changed from nitrate to nitrate_plus_nitrite.

September 11, 2001:

Changed Oxygen (mL/L) to Oxygen:dissolved:volume and added Oxygen:dissolved:mass (umol/kg) channel.

# February 13, 2002 – Joe Linguanti

Started work from Melanie’s directory \\OSAPCOMMON\STPCHEM\FINISHED\1969\6902.

References:

1. Fisheries Research Board of Canada (FRB), Technical Report No. 145, Oceanographic Observations At Ocean Station P, (50N, 145W), 21 February to 9 April 1969.

2. Stephens, K. 1970. Data record : Primary Production Data from the Northeast Pacific Ocean January 1967 to December 1969. Fisheries Research Board of Canada Manuscript report series No. 1123.

- Cast numbers were renamed to be the same as the event numbers for both bottle and CTD files.

- Program HEADER EDIT was used to change non-standard channel names, units, Data Description, Scientists name, Project and Platform.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

## April 4, 2002 – Joe Linguanti

Reference:

Stephens, K. 1977. Primary Productivity Data from Weatherships Occupying Ocean Station "P" 1969 to 1975. Fisheries & Marine Service Data Report No. 38.

Three productivity casts (9000 series) containing chlorophyll, and nitrate_plus_nitrite data was from Jon Page’s files. The primary production data was obtained from the report cited in reference. File processed in February containing only the Nitrate_plus_Nitrite data were removed from the archive.

NOTE for cast 9002:

Nitrate data obtained from "Stephens K., MS, 1970. Data Record. Primary Production Data from the N.E. Pacific Ocean, January 1968 to July 1970. Fisheries Research Board of Canada, Manuscript Report Series, No. 1123".

Productivity data differs significantly from the 1970 and 1977 data reports.