## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 14-Jan-2011 | Added nutrient data below 250 metres to nutrient cast(s) obtained from John Page in 2002. In 2002 John was under the assumption that all nutrient values below the mixed layer were questionable so he did not include any values greater than 250 metres. In 2006 John found out that this assumption applied only to Silicate data, so he put back the deep values for Nitrate and Phosphate into his database. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\. |

# February 27, 2002 – Joe Linguanti

References:

1. Oceanographic Observations at Ocean Station P (50N, 145W), Vol.52 August 6, 1971 – January 16, 1972,  Pacific Marine Science Report 72-2.

2. Stephens, K. 1977, Primary Productivity Data from Weatherships Occupying Ocean Station "P" 1969 to 1975. Fisheries & Marine Service Data Report 38.

- Three bottle casts 5, 11 and 41, which were not in Sus’s database, were obtained from the data report cited in reference 1.

- Four productivity casts (9000 series) containing nutrient and chlorophyll data were obtained from John Page’s files.

- One cast containing nutrient data was obtained from John Page’s files and was merged with the matching hydro files from Sus’s database since they were sampled at the same time.

John’s file 1 merged with hydro file 16.

- Oxygen:Dissolved:Mass was added, using program CHGUNITS, to all bottle files with oxygen data.

- No CTD salinity data since difficulties were experienced in salinity trace adjustments. Terminal on salinity sensor was difficult to waterproof.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

## April 23, 2002 – Joe Linguanti

Reference:

1. Stephens, K. 1977. Primary Productivity Data from Weatherships Occupying Ocean Station "P" 1969 to 1975. Fisheries & Marine Service Data Report No. 38.

Primary production data was added to three files previously obtained from John Page (9000 series). The primary production data was obtained from the report cited in reference 1.

## April 24, 2002 – Joe Linguanti

CTD cast 39 - Day in Start time changed from 19 to 20.

CTD cast 49 - Station changed from 3 to 2.

- Longitude changed from 126 40.0 to 126 0.0