## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 17-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 17-Feb-2011 | Added a new cast 9001 containing Tritium data from John’s spreadsheet file and added one tritium value to the surface Loop file.. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\Tritium. |
| 02-Apr-2010 | Added Loop data from the publications referenced below. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

# May 28, 2002 – Joe Linguanti

References:

1. Oceanographic Observations at Ocean Station P (50N, 145W), Vol. 83,  June 17 – August 3, 1977,  Pacific Marine Science Report 77-24.

- Oxygen:Dissolved:Mass was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields. NOTE: In Sus’s files the data type field in the header contained a “B” for a Bissett-Berman CTD, therefore DEPTH was in the file. The data report specifies that a Guildline STP was used, hence the DEPTH channel was renamed to PRESSURE.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.