## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 02-Apr-2010 | Added Loop data from the publications referenced below. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

# May 30, 2002 – Joe Linguanti

References:

1. Oceanographic Observations at Ocean Station P (50N, 145W), Vol. 95,  October 20 – December 6, 1978,  Pacific Marine Science Report 79-9.

- No bottle data for this cruise.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

NOTE: The data values in Sus’s data files differ from the values in the data report because a second set of corrections was applied in September 1979.

## Notes from Joe Linguanti on September 19, 1979

No hydro data was taken on cruise 78-8, therefore the comparisons were done using the last three hydro stations from 78-7 and comparing them with the first three Station P CTP stations from 78-8. The differences were calculated, plotted and the regression line was then determined. The corrections were applied to the uncorrected data.

The Temperature correction was done in two parts,

1)	     0 metres 	+0.35

200 metres	+0.19

2)	 200 meters	+0.19

1500 meters	+0.32

The Salinity correction was done in two parts,

1)	      0 meters	-0.19

200 meters	-0.14

2)	  200 metres	-0.14

1500 metres	-0.10

The data in the upper 200 metres were widely scattered therefore the differences at the surface were averaged and a regression line was applied from the surface to 200 metres.