## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 02-Apr-2010 | Added Loop data from the publications referenced below. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

# May 30, 2002 – Joe Linguanti

References:

1. Oceanographic Observations at Ocean Station P (50N, 145W), Vol. 96,  December 1, 1978 – January 10, 1979,  Pacific Marine Science Report 79-10.

- No bottle data for this cruise.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

NOTE: The data values in Sus’s data files differ from the values in the data report because a second set of corrections was applied in September 1979.

## Notes from Joe Linguanti on September 24, 1979

No hydro data was taken on cruise 78-9, therefore the comparisons were done using the first three hydro stations from 79-2 and comparing them with the last three Station P CTP stations from 78-9. The differences, both at the surface and from 100 to 1200 metres, were calculated, plotted. The surface temperature differences were calculated using the raw CTP temperatures against the surface temperatures in the MET log for 78-9. The salinity differences were calculated using the raw CTP values at 10 metres against the seawater loop measurements for 78-9. The corrections were applied to the uncorrected data.

The Temperature correction is,

1)	      0 metres 	+0.03

1500 metres	-0.28

The Salinity correction is,

1)	      0 meters	+0.15

1500 meters	+0.61