## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 17-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 11-Apr-2010 | Added Loop data from the publications referenced below. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. The time was missing from the first 4 stations so I used times based on the sailing times between stations from a previous cruise. |

# June 4, 2002 – Joe Linguanti

References:

1. Oceanographic Observations at Ocean Station P (50N, 145W), Vol. 102,  July 27 – September 13, 1979,  Pacific Marine Science Report 81-16.

- Oxygen:Dissolved:Mass was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.