| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 18-Feb-2011 | Added nutrient data below 250 metres to nutrient cast(s) obtained from John Page in 2002. In 2002 John was under the assumption that all nutrient values below the mixed layer were questionable so he did not include any values greater than 250 metres. In 2006 John found out that this assumption applied only to Silicate data, so he put back the deep values for Nitrate and Phosphate into his database. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\. |
| 24-Apr-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 20, 1993

Cruise 81-05

Aug 18 - 28 1981

61 processed files.

casts 1 - 78

This data was collected using the HP 2100 acquisition system.

NOTE:

-----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  Hydro corrections were applied

New_Temperature = Old_Temperature + ( 0.001963*Old_Temperature - 0.072471)

New_Salinity    = Old_Salinity    + (-0.017822*Old_Temperature + 0.325567)

Joe Linguanti

John Peart's processing notes follow

------------------------------------

LOG OF PROCESSING FOR WEATHERSHIP TAPES

'81-MS-1'

1/   WHEN CONVERTING THE HP TAPES TO UNIVAC FORMAT, PARITY

ERRORS WERE ENCOUNTERED ON THE FOLLOWING

FILES:

FILE  CONSECUTIVE #  TAPE

---------------------------

37        49          1

38        50          1

5        63          2

THE PARITY ERRORS APPEARED TO BE ENCOUNTERED ON THE LAST

RECORD OF EACH FILE.

2/   A CHECK AGAINST THE CRUISE LOG SHOWED THAT THE FILES FOR

CONSECUTIVE NUMBERS 57 AND 58 WERE MISSING FROM THE HP

TAPES AND THEREFORE NEED TO BE DIGITIZED.

3/   BASED ON THE CRUISE LOG THE FOLLOWING CHANGES WERE

MADE:

A/  FILES 1, 2 AND 3 WERE DELETED BECAUSE THEY APPEARED

TO BE TEST RUNS ONLY

B/  FILES 49 AND 57 WERE DELETED BECAUSE THEY APPEARED

TO BE ABORTED CASTS THAT WERE REPEATED IN THE

FILES 50 AND 58 RESPECTIVELY.

C/  THE STATION NUMBERS WERE CHANGED AS FOLLOWS:

CONSEC#   FILE      OLD #    NEW #

1       4         1       P001

3       5         2       P002

4       6         3       P003

5       7         4       P004

7       8         5       P005

13      14        LP06     P006

19      18        LP10     P010

45      35        P26E     E026

49      37        R33      P033

62      48        J O1     J001

75      63        -04      P004

D/   LONGITUDES WERE CHANGED AS FOLLOWS:

CONSECUTIVE #   FILE     OLD LONG   NEW LONG

43           33       133363     143363

59           45       123240     133240

61           47       132500     131500

4/  THE 61 CASTS WERE THEN RUN THROUGH THE DELETE

PROGRAMME WHICH DISCARDS ALL RECORDS WHICH

DO NOT CONTINUE A SEQUENCE OF INCREASING DEPTH.

5/  PLOTS WERE THEN GENERATED OF ALL CASTS.  AREAS OF

ANAMOLOUS TEMPERATURE AND SALINITY READINGS

WERE ISOLATED AND DELETED FROM THE FILES AS

FOLLOWS:

CONSEC #     RECORDS DELETED

1         1-3, 6-17, 23-27

3         1-18, 20-41, 43-58, 60-64, 75-82

4         40-46

5         1 -11, 14-19, 22, 46-49, 58-60

7         1 - 8, 11-15, 19-20, 22-28, 30-39,

77, 80, 91-92, 102-107, 1079, 1885-1886

8         1 - 8, 16-19, 24, 27, 31-35, 39-40

9         110-114, 119-124, 138-140

10         35-40, 42-43

11         1 -26, 45-48, 51, 54, 58-63

13         83-89, 146-147, 152-153

15         32-33, 35-38, 42-45, 47-50, 54-58

16         23, 33-39, 271-275, 302-306

18         1 - 8, 14-15, 284, 287-288, 291-294,

300, 303-305, 318

19         1 - 4, 84, 89, 104, 114

21         1 -17, 263, 276, 279-284, 310-314, 316-319,

321-326, 392-395, 2124-2125

22         181, 184, 186, 189-194, 275-281

24         152-161, 163-165, 167, 229-233

250, 252,255, 257-259, 261-262

26         97, 99, 101, 125-127, 185-186, 188,

190-191, 196-205

28         180, 186-189, 191, 193-196, 214-216

29         116-117, 121-122, 124-126, 133-134,

141-145, 157, 159-161, 168-169

31         98-102, 106-107, 114, 118-122, 125-126,

137-138, 146-151, 158-161, 163-165

167-168, 171, 174, 210-217, 219-220, 222

32         101, 103, 106, 108-114

34         105-110, 112-115

35         120-126, 130-132, 136, 142

37         125-130, 132-136, 139-140, 155-157

38         120, 122-131, 134

40         159-163, 165-170, 172-175, 177-178

41         173-174, 176, 178-183, 200-201

43         176-180, 182-184

44         147, 149-154, 156-161, 164-165, 167

46         1-2, 4, 6-16, 18-22, 157, 173-174, 180-183,

185, 190-192, 197-201, 294-296, 353-358

50         7-8, 131-134, 142-144, 150-151, 153-158

160-164

51         94, 100, 104, 108, 113, 118-121

52         117-121, 154-157

53         122, 124-125, 127-128, 130-133, 145-146,

163-165, 192-193, 3367

54         139-144

55         141, 149-153, 156-158, 162, 164, 170-171,

177-179, 243, 245-254

56         9-17, 21, 23, 25, 32-33, 120-124, 127, 129,

153-155

59         67-69, 108-109, 114-116, 119, 143-146,

2939-2940

60         1878-1924

62         129, 138, 203-205, 216-220, 222, 226-229

538, 548-551, 554-556

63         838-841

64         208-211, 223-236, 238

66         104-106

68         227-228

69         1-6, 8-9, 2321

70         62-63, 76-79

71         233-235, 259-273

72         20-27, 140-143, 224, 285-288

73         23, 35, 40-41, 84-98, 100-106, 108-109,

131-138, 165-168, 977

74         1-19, 24-26, 97-114

75         7, 10-14, 16-20, 24, 27-28, 30, 32-33,

36-39, 41-46, 110, 113-118, 132-135,

1013

76         1-8, 10-17, 21, 32-38, 42-43, 83-85, 90,

93-98, 100-107, 110, 225, 257-261,

267-280

77         1-17, 27-28, 33-47, 53, 56, 60, 63-69, 71

78         1-12, 50-55, 69-73

6/  THE 61 CASTS WERE THEN PROCESSED WITH THE ONE METRE

AVERAGING PROGRAMME AND THEN PASSED THROUGH THE

SMOOTHING PROGRAMME WHICH DOES ONE FINAL CHECK FOR

TEMPERATURE AND SALINITY SPIKES.  THE OUTPUT

FROM THE SMOOTHING PROGRAMME IS STORED ON TAPE #

U00227 WHICH WILL BE SAVED FOR BACKUP PURPOSES.

7/  COMPARISON RUNS WERE THEN MADE AGAINST THE HYDROCAST

DATA.  THE FACTORS USED IN THE RUNS WERE:

1 => DEPTH VS TEMP,SALINITY - ALL DATA

2 =>         ''             - DEPTHS 21-299 EXCLUDED

3 =>         ''             - DEPTHS 0 -299 EXCLUDED

4 => TEMP VS TEMP,SALINITY  - ALL DATA

5 =>         ''             - DEPTHS 21-299 EXCLUDED

THE BEST FIT APPEARED TO BE MADE ON RUN # 5.  THE

REGRESSION PLOT FROM THIS RUN IS SAVED IN FILE

'PL81-MS-1'.

8/  ALL THE FILES WERE THEN PROCESSED USING THE TEMPERATURE

CORRECTION PROGRAMME AND THE SLOPE FORMULAE FROM

COMPARISON RUN # 5.

TCORR = (T * 0.00198271) - 0.0724714 + T

SCORR = (TCORR * (-0.017822)) + 0.325567 + S

THE CORRECTED DATA IS STORED ON TAPE 81-MS-1, #U01047.

# Moving files to new archive. June 10, 2002 – Joe Linguanti

- There was no hydro files in the OSAP data archive so I copied the STDHYD_81.DAT file from one of Kerry Kinnersley’s tapes and converted it to the 12-line format, used in the 1980s. The 12-line header files were then converted to IOS Header files.

- One cast containing nutrient data was obtained from John Page’s files. John’s file 1 was merged with bottle file 47.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.