| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 18-Feb-2011 | Added nutrient data below 250 metres to nutrient cast(s) obtained from John Page in 2002. In 2002 John was under the assumption that all nutrient values below the mixed layer were questionable so he did not include any values greater than 250 metres. In 2006 John found out that this assumption applied only to Silicate data, so he put back the deep values for Nitrate and Phosphate into his database. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\. |
| 24-Apr-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           June 21, 1993

Cruise 81-06

Oct 20 - 27 1981

53 processed files.

casts 1 - 62

This data was collected using the HP 2100 acquisition system.

NOTE:

-----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  Hydro corrections were applied

For casts 1 - 38

New_Temperature = Old_Temperature + (-0.004165*Old_Temperature + 0.057779)

New_Salinity    = Old_Salinity    + ( 0.005210*Old_Temperature - 0.076733)

For casts 39 - 62

New_Temperature = Old_Temperature + (-0.004165*Old_Temperature + 0.057779)

New_Salinity    = Old_Salinity    + ( 0.002658*Old_Temperature - 0.050017)

Joe Linguanti

John Peart's processing notes follow

------------------------------------

PROCESSING SEQUENCE FOR WEATHERSHIP CRUISE

81-MS-2,   DECEMBER 1982 BY J. PEART

1/  A DUMP OF THE HP TAPE HEADERS REVEALED THAT CASTS 1,2 & 4

WERE NOT RECORDED.

2/  TAPES WERE TRANSLATED AND LOADED ONTO ONE UNIVAC TAPE

3/  ON THE 'DELETE' RUN  CASTS 6 AND 8 WHICH HAD BEEN ABORTED WERE

DELETED AS WELL AS THE FIRST FILE FROM THE SECOND HP REEL

WHICH WAS LABELLED TEST.

ON THIS RUN CASTS 36,37,AND 38 (LABELLED BUOY 1,2&3) WERE

RENAMED ($ 1, $ 2, & $ 3).  THESE WERE CASTS TAKEN WHEN BUOYS

WERE RELEASED FOR THE US NATIONAL CENTER FOR ATMOSPHERIC

RESEARCH IN BOULDER COLORADO.  THEY WERE NOT TAKEN AT

USUAL STATIONS.   ALSO THE CONSEC # '512' WAS CORRECTED TO '51'

4/    RAW PLOTS WERE THEN MADE OF EACH CAST.  SLIP RING

PROBLEMS WERE EXCESSIVE ON THIS CRUISE WHICH MADE THE

USUAL EDITTING PROGRAMME UNUSABLE FOR MOST CASTS.

SALINITY SPIKES WERE REMOVED BY MAKING A ROUGH MEASURE-

MENT OF SLOPE TRENDS IN SET PRESSURE REGIONS.  THESE

SLOPE FACTORS WITH A GIVEN DEGREE OF ACCEPTANCE WERE

THEN FED TO THE PROGRAMME 'EDSLOPE' WHICH PROCESSED

EACH CAST AND DISCARDED DATA WHICH WAS OUTSIDE THE REGION

OF ACCEPTANCE.  A LISTING OF DISCARDED DATA IS CONTAINED

IN THE DOCUMENTATION FILE.

5/   ALL CASTS WERE THEN REPLOTTED, LOADED ONTO DISK AND

ANY REMAINING SPIKES WERE THEN DELETED MANUALLY.  NO

RECORD OF THIS DELETED DATA WAS MAINTAINED.

6/   AT THIS POINT CASTS 1, 2 AND 4 WERE TYPED INTO DISK FILES

USING THE PRINT LOG FROM THE HP AS INPUT.  ANY SPIKES

WERE EDITTED OUT.

7/   ALL 53 CASTS WERE THEN UNLOADED BACK TO TAPE WHERE THEY

WERE PROCESSED THROUGH THE 1 METRE AVERAGING AND SMOOTHING

PROGRAMMES.

8/   BASED ON PRELIMINARY WORK DONE BY S. TABATA, IT WAS

DETERMINED THAT A SUDDEN CHANGE IN SALINITY READINGS

HAD OCCURRED AFTER THE 'NCAR' BUOYS HAD BEEN DROPPED.

WHEN THE HYDROCAST - STD CORRELATION RUNS WERE MADE

THE FOLLOWING CONTROL FACTORS WERE IN EFFECT:

A/ ALL HYDROCASTS WERE USED IN THE TEMPERATURE CORRELATIONS

BUT DATA IN THE 50 TO 100 DB RANGE WAS NOT USED.

B/ HYDROCASTS 17,22,27,32 AND 35 WERE USED IN THE SALINITY

CORRELATIONS FOR STD CASTS 1 TO 38.  DATA IN THE 50 - 150

DB RANGE WAS NOT INCLUDED.

C/ ONLY HYDROCAST 52 WAS USED TO DETERMINE THE SALINITY

CORRELATION FOR STD CASTS 39 TO 62.

9/ WITH THE SLOPE AND INTERCEPT VALUES CALCULATED ON THE

CORRELATION RUNS THE FOLLOWING CORRECTIONS WERE

APPLIED:

CASTS 1 TO 38

TCORR = (T*(-O.OO4165)) + 0.05779 + T

SCORR = (TCORR*0.005210) - 0.076733 + S

CASTS 39 TO 62

TCORR = (T*(-O.OO4165)) + 0.05779 + T

SCORR = (TCORR*0.002658) - 0.050017 + S

10/  THE CORRECTED DATA WAS STORED ON TAPE '81-MS-2', #U00402.

THE BACKUP TAPE WAS BROUGHT BACK TO THE OFFSHORE TAPE

ROOM AND PHYSICALLY LABELLED AS 'BACKUP TO U00402'.

# Moving files to new archive. June 10, 2002 – Joe Linguanti

- There was no hydro files in the OSAP data archive so I copied the STDHYD_81.DAT file from one of Kerry Kinnersley’s tapes and converted it to the 12-line format, used in the 1980s. The 12-line header files were then converted to IOS Header files.

- One cast containing nutrient data was obtained from John Page’s files. John’s file 1 was merged with bottle file 35.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.