| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 7-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 18-Feb-2011 | Added nutrient data below 250 metres to nutrient cast(s) obtained from John Page in 2002. In 2002 John was under the assumption that all nutrient values below the mixed layer were questionable so he did not include any values greater than 250 metres. In 2006 John found out that this assumption applied only to Silicate data, so he put back the deep values for Nitrate and Phosphate into his database. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\. |
| 01-May-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 22, 1993

Cruise 82-03

May  4 - 14 1982

60 processed files.

casts 1 - 72

This data was collected using the HP 2100 acquisition system.

NOTE

----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  applied lab pressure calibrations to pressures > 100 dbars.

new_press = 0.02583*old_pressure - 0.23691

2.  Hydro corrections were applied

New_Temperature = Old_Temperature + ( 0.006759*Old_Temperature - 0.054054)

New_Salinity    = Old_Salinity    + (-0.004617*Old_Temperature - 0.005409)

Joe Linguanti

----------------------------------------------------------------------

----------------------------------------------------------------------

----------------------------------------------------------------------

Oct 28/96  Robin Brown

These data compared with STNP archives - station locations and data

match.  STNP archive CTD profiles are decimated, so only HYD files

were added to IOS archive.  It would appear that HYD files have a

unique consecutive number. 12 HYD files added to archive.

# Moving files to new archive. June 11, 2002 – Joe Linguanti

- One cast containing nutrient data was obtained from John Page’s files. John’s file 1 was merged with bottle file 36.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.