| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 7-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 08-May-2010 | Added TCO2 data for cast 1982-05-0039.BOT from C.S. Wong’s archive. |
| 01-May-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 22, 1993

Cruise 82-05

Sep 17 - 30 1982

57 processed files.

casts 1 - 72

This data was collected using the HP 2100 acquisition system.

NOTE

----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  Hydro corrections were applied

Casts  1 - 64 and 70 -72

New_Temperature = Old_Temp + ( 0.000303*old_Temp - 0.001845)

New_Salinity    = Old_Sal  + (-0.006906*Old_Temp + 0.081663)

Casts 65 - 69

New_Temperature = Old_Temp + (-0.00017*old_Temp + 0.33)

New_Salinity    = Old_Sal  + (-0.00015*Old_Temp - 0.07)

Joe Linguanti

----------------------------------------------------------------------

Oct 28/96 Robin Brown

HYD files (11) added from STN P archive. Note that HYD file names are

not consistent with CONSEC numbers.

----------------------------------------------------------------------

# Moving files to new archive. June 11, 2002 – Joe Linguanti

- There was no hydro files in the OSAP data archive so I copied the STDHYD_82.DAT file from one of Kerry Kinnersley’s tapes and converted it to the 12-line format, used in the 1980s. The 12-line header files were then converted to IOS Header files.

- One cast containing nutrient data was obtained from John Page’s files. John’s file 1 was merged with bottle file 39.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.