| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 01-May-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 23, 1993

Cruise 84-02

Aug 21 - 31 1984

31 processed files.

casts 1 - 38

This data was collected using the HP 2100 acquisition system.

NOTE

----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  Particulars

Cast numbers 16,29,35,36,37 and 38 were digitized from analog traces

since computer crashed during these casts and files were not on tape.

Nearly all salinity profiles were quite noisey in the upper 80 dbars.

2.  Hydro corrections were applied.

New_temperature = Old_temp + ( 0.003206*Old_Temp - 0.051716)

New_Salinity    = Old_Sal  + ( 0.001090*Old_Temp + 0.019743)

Joe Linguanti

-----------------------------------

Oct 30/96 Robin Brown

IOS archive CTD's (31) reconciled with STNP archive CTD's.7 HYD files

added from Station P archive.  These HYDRO files match the files in

the [HYDRO] directory for cruise 84M2.

----------------------------------------------------------------------

# Moving files to new archive. June 13, 2002 – Joe Linguanti

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.

March 25, 2004

Cast 29.CTD - changed position from 49 50.20, 142 40.00 to 50 00.00, 143 36.30.

Cast 21.BOT - changed latitude from 49 26.00 to 49 16.90.