| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 29-Mar-2017 | Renamed fluorometer channel name from "Fluorescence:URU" to "Fluorescence:Calibrated" since the data from the fluorometer (on the CTD) has been converted to Chlorophyll-a using linear regression on observed extracted chl-a. Joe Linguanti. |
| 23-Jan-2017 | Recalibrated PAR by adding +2 based on dark value; PAR and PAR:Reference removed from casts 35 and 37-40 as it appears not to have been mounted on the CTD because it went too deep. |
| 01-May-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Nov  30, 1993

Cruise 84-30

May 12 - 24 1984

40 processed files.

casts 1 - 40

This was Ocean Ecology cruise 84-02.

NOTE

----

Consecutive numbers are not in chronological order.

Joe Linguanti

----------------------------------------------------------------------

Oct 31/96 Robin Brown

IOS Archive CTD files (40) reconciled with STN P archive (40). No

HYDRO files were found in the Station P archive.  I edited some

profiles to remove spikes at the thermocline at surface.

-------------------------------------

# June 18, 2002 – Joe Linguanti

References:

Ocean Ecology Data Report: Subarctic Pacific Ocean, May 1984 (Project SUPER). 1988. Canadian Data Report of Hydrography and Ocean Sciences, No. 64.

- Crossed-referenced John’s nutrient files with the bottle files from the archive. The data in John’s files were contained in the archive files so John’s files were not used. Bottle files 3, 4, 7, 9, 11, 14, 21, 22, 23 and 25 contained only CTD data so they were not archive as CTD files at approximately the same time were available.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.