| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 15-May-2010 | Added Loop data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 23, 1993

Cruise 85-01

Feb 12 - 15 1985

12 processed files.

casts 1 - 15

This data was collected using the HP 2100 acquisition system.

NOTE

----

The meter average program replaced missing surface records with the

first available records. For example: if the first record in the file

started at 4.0 dbars then the program would create records at 0, 1, 2

and 3 dbars with identical temperature, salinity, conductivity  and

transmissivity values of those at 4 dbars.

1.  Particulars

Since salinity was extremely noisy the data was averaged into 5 meter

bins before editing.

cast 8 deleted since temperature and salinity were bad.

No corrections applied since comparison with hydro data was very good.

Joe Linguanti

-----------------------------

Oct 31/96 Robin Brown

IOS archive CTD (12) reconciled with STN P archive CTD's (12). One

hydro file (equivalent to HYDRO85M1.DAT) was added to IOS archive.

----------------------------------------------------------------------

# Moving files to new archive. June 21, 2002 – Joe Linguanti

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files *.CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.CHE “.