| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 25-Feb-2011 | Added Dissolved Inorganic Carbon (DIC) data to bottle casts 1 and 6 from Jim Christian’s DIC file located in the \DOC directory. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\Alkalinity and DIC. |
| 06-Jan-2011 | Added C13 (cast 4) profile data from C.S. Wong’s archive. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |

Processing notes           Jun 25, 1993

Cruise 86-03

Oct 15 - 29 1986

52 processed files.

casts 1 - 52

This data was collected using the IBM AT (program DCTD.BAS) acquisition system.

1.  Particulars

Cast 1 - salinity from 0 to 7 dbars replaced with value at 7 dbars

Cast 2 - salinity from 0 to 16 dbars replaced with value at 16 dbars

1.  Hydro corrections were applied.

New_temperature = Old_temp + ( 0.00270*Old_Temp - 0.03057)

New_Salinity    = Old_Sal  + (-0.00104*Old_Temp - 0.00467)

Joe Linguanti

----------------------------------------------------------------------

----------------------------------------------------------------------

Nov 8, 1996 Robin Brown

IOS archive CTDs(52) reconciled with STNP archive CTDs (52). 8 HYD

files added from STNP archive (c.f. HYDRO86M3.dat).

----------------------------------------------------------------------

----------------------------------------------------------------------

# Moving files to new archive. June 26, 2002 – Joe Linguanti

- Three files containing nutrient data was obtained from John’s files of which only one was merged with a hydro cast file. Nutrient cast 2 and was merged with hydro cast 32. The other 2 nutrient files were renumbered to 9001 and 9002.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

May 25, 2003 – Joe Linguanti

Added loop file to the archive. This file was produced by Bernard Minkley from older text files.