| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 23 July 2012 | Phosphate was recalibrated by subtracting 0.10 uM. See details below. |
| 16-May-2010 | Added productivity, POC and PON data from John Page’s spreadsheet to 1 productivity profile and DIC to 3 nutrient profiles. Further metadata and Data Sources can be found in John Page’s spreadsheet file in directory \\OSD_Data_Archive\Cruise_Data\Documents\John_Pages_Files\. |

July 23, 2012

Frank Whitney recommended this correction to the PO4 data to remove a bias in the Line P time series data caused by an incorrect zero (fresh water was used for the baseline during analyses instead of salt water, thus the baseline was too low by 0.10 uM).

The amount of the correction was determined by observing the offset in the PO4 time-series at Station P.

# Moving files to new archive. July 5, 2002 – Joe Linguanti

- Three hydro files containing nutrient data were obtained from John’s files and merged with the hydro casts from the archive. One productivity file was obtained from John's files (9000 series).

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

- Line P cruises in 1987 should all have PO4 reduced by 0.10 uM. (cruises 87-02 to 87-05 in Chemistry notation).