## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 11-Jan-2011 | Added DIC from C.S. Wong’s archive to 3 bottle cast 1, 2 and 3 and one productivity cast 9001. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |
| 10-Jan-2011 | Added Temperature, CTD Salinity and DIC from C.S. Wong’s archive to Loop data. Data sources and further metadata can be found in John Page’s spreadsheet file located in the \DOC directory. |
| 14-Dec-2004 | Made corrections to loop file. See December 14, 2004 notes below. J.L. |

Processing notes           Aug 14, 1993

Cruise 88-03

Nov 30 - Dec 12 1988

61 processed files.

casts 1 - 61

This data was collected using the IBM AT (program DCTD.BAS) acquisition system.

1.  Particulars

- Surface offset of -4.0 was added to all pressures.

2.  Hydro correction was applied only to salinity.

New_Salinity    = Old_Sal  + (-0.117249E-04*Depth - 0.133678E-01)

Joe Linguanti

---------------------------------------------------------------------------

Nov 12/96 Robin Brown

IOS archice CTD's (61) reconciled with STNP archive (61). 5 HYD files added

from STNP archive.

# Moving files to new archive. July 11, 2002 – Joe Linguanti

- Four hydro files containing nutrient data were obtained from John’s files and merged with the hydro casts from the archive.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

May 22, 2003 – Joe Linguanti

- The loop file was obtained from Frank Whitney. The spreadsheet file was converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory.

December 14, 2004 – John Page’s email

Hi Joe.   The Dates are a little off in files    1988-03-surface.loop     and    8803loop.xls   in OSAP Cruisedata archive.

It appears that the LineP data was cobbled together from outbound and inbound segments to appear as a continous outbound sequence.

The dates for Stns MP18, MP19, MP20, MP21  should be Dec 08

The dates for Stns MP23. MP24, MP25, MP35  should be Dec 07

... deduced from looking at 1988-03-*.ctd files.

Attached is the surface T,S data extracted from the .CTD files for comparison   'surf-ctd-1988-03-03.txt'

and my own Excel file 8803loop-debug.xls    to show where the problems are in archive file '8803loop.xls'  Suggested correction to far right.