| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 10-Jan-2014 | Added underway pCO2 data from Sophia Johannessen’s Excel files prepared for The Carbon Dioxide Information Analysis Center (CDIAC). The file is located in the cruise .DOC directory. |
| 30-Jan-2012 | - Merged Primary Productivity (PP), POC and PON from Frank Whitney’s Productivity spreadsheet to 4 productivity casts. The PP in units of mgC/m^3/hr already in the 4 productivity casts were replaced with the PP values in units of mgC/m^3/day from Whitney’s spreadsheet file. - Merged DIC data to bottle cast numbers 1, 3, 5, 6, 7, 8, 9 and 11 from Jim Christian’s DIC file located in the \DOC directory. For more information, including quality control analysis see John Page’s spreadsheet files in directory \\OSD_Data_Archive\Cruise_Data\Documents\Station Papa Data Archiving\Alkalinity and DIC. |

Processing notes           Aug 15, 1993

Cruise 90-01

May 10 - 29 1990

98 processed files.

casts 1 - 113

This data was collected using the HPVECTRA 386 (program CTD) acquisition system.

1.  Particulars

- Pressure offset of -4.0 dbars was added to all pressures.

- still a few bugs in new acquisition program therefore alot of casts

were aborted and redone. These are 4,6,8,23,26,33,35,47,48,57,59,60,

66,67 and 89.

- Casts 34,36,45,52,61,63 and 92 were upcasts.

- Cast 44 - transmissivity deleted since it was questionable. Cast 45

which is the upcast contains transmissivity.

- Cast 56 - deleted alot of records from CNV file since data records

were garbage.

- Cast 88 - Salinity unusually noisy. An attempt was made to remove as

many spikes as possible

2.  Hydro corrections were applied.

New_temperature = Old_temp + (-0.717588E-05*Pressure + 0.202912E-01)

New_Salinity    = Old_Sal  + (-0.212877E-05*Pressure - 0.296414E-01)

Joe Linguanti

----------------------------------------------------------------------------

Nov 12, 1996  Robin Brown

IOS ARCHIVE CTDS (98) reconciled with STNP archive CTD's (98). 12 HYD files

added

from STNP archive - 12 90-01-xxx.HYD files moved to [ctd/hydro]

# Moving files to new archive. July 15, 2002 – Joe Linguanti

- Eight hydro files containing nutrient data were obtained from John’s files and merged with the hydro casts from the archive. Four productivity casts (9000 series) were obtained from John’s files.

- Oxygen:Dissolved in umol/kg was added, using program CHGUNITS, to all bottle files with oxygen data.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

May 22, 2003 – Joe Linguanti

- The loop file was obtained from Frank Whitney. The spreadsheet file was converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory.

- The SAIL files were copied from the TRACKOBS directory in the OSAP Data Archive.

- Channels not needed were removed.

- Program HEADER EDIT was used to change non-standard channel names and units and add additional metadata to the files.

- IOS Header Date and Time channels were added.

- Program Clean was used to set the start and end times and the geographical area.