Processing notes           Aug 21, 1993

Cruise 91-08

Apr 29 - May 9 1991

78 processed files.

casts 1 - 80

This data was collected using the HPVECTRA 386 (program CTD) acquisition system.

1.  Particulars

- Two CTD probes were used on this cruise.

casts  1 - 21 probe 53,501

casts 22 - 80 probe 58,483

- Pressure offset

for probe 53,501 -4.0 dbars was added to all pressures.

for probe 58,483 -5.0 dbars was added to all pressures.

- Salinity data had alot of small spikes due to large swells and huge

temperature fluctuations. As much as possible were edited by digitizing

through them, however some of the smaller spikes were left in.

- Cast 19 - aborted cast

- Cast 20 - removed structure in salinity in the upper 25 dbars since

we think it was caused as a result of the CTD malfunctioning.

- Cast 21 - aborted cast

2.  Hydro corrections were applied.

Casts  1 - 20

New_temperature = Old_temp + ( 0.243225E-04*Pressure + 0.163760E-01)

New_Salinity    = Old_Sal  + (-0.201243E-05*Pressure - 0.111331E-01)

Casts 22 - 80

New_temperature = Old_temp + (-0.201114E-04*Pressure + 0.251486E-01)

New_Salinity    = Old_Sal  + ( 0.762268E-05*Pressure - 0.953614E-02)

Joe Linguanti

October 18, 2001. Joe Linguanti

--------------------------------

File 91080030.che - changed latitude from 48 to 49.

All Files - changed chief scientist from Freeland to Tabata.

# Moving files to new archive. July 23, 2002 – Joe Linguanti

- Ten hydro files containing temperature, salinity, oxygen and nutrient data were obtained from the OSAP archives. Bernard had merged the nutrient data from Frank Whitney’s data set in 1999.

- Program HEADER EDIT was used to change non-standard channel names and units and Data Description, Scientists name, Project, Platform and Instrument fields.

- Program Clean was used to;

- reset number of records, minimum and maximum values

- delete empty channels where applicable

- set channel name initial letters to uppercase

- Set last 4 characters of file name to event – set cast numbers to be the same as event numbers in file.

- Files .CLN were copied to new archive and renamed to the new file convention “YYYY-CC-NNNN.* “.

May 21, 2003 – Joe Linguanti

- The loop file was obtained from Frank Whitney. The spreadsheet file was converted to IOS Header format with meta-data added to it. The original .XLS file is in the DOC directory.

May 4, 2016 – Roy Hourston

IOS Shell CLEAN function performed on some BOTTLE files to fix wrong formats.