July 17, 1992

Current State of 9207 CTD Processing

All casts have been processed as follows:

·	.CNV files created from .HEX files (all header information checked and corrected as necessary)

·	Shift T in .CNV files by -2 records

·	Recalc Sal in .SHF

·	Shift C in .RCL files by -.5

·	Clean .SHF files by removing all up casts, subtracting min pressure from all pressure values, and removing top 2 meters of data from all casts.

·	Delete ran on .CLN files to extract first down cast.

NOTE:Cast 62 should be checked to make sure that second rather than first down cast was used.

·	1/2 meter average files created from .DEL files

·	I have gone through every cast and determined which ones are good and which ones suffer from pumping problems.  I have then determined which cast is to be used for each station.  They are as follows

| Station | Cast# | Station | Cast# | Station | Cast# |
| --- | --- | --- | --- | --- | --- |
| E100 | 45 | E400 | 46 | E700 | 20 |
| E110 | 33 | E500 | 15 | E800 | 49 |
| E200 | 51 | E600 | 42 | E900 | 30 |
| E210 | 36 | E620 | 41 | MB92 | 70 |
| E220 | 32 | E630 | 40 | R192 | 54 |
| E225 | 62 | E640 | 39 | R292 | 55 |
| E230 | 31 | E650 | 38 | SITL | 69 |
| E300 | 72 | E660 | 37 | T100 | 25 |
| E330 | 61 | E661 | 64 | T200 | 24 |
| E341 | 60 | E662 | 65 |  |  |
| E350 | 59 | E663 | 53 |  |  |
| E360 | 58 | E665 | 52 |  |  |
| E370 | 57 | E670 | 66 |  |  |

·	I went through each bottle cast salinity and determined the comparitive CTD salinity for that station and depth.  I then noted what were valid bottles for intercomparisson based on the variability of the water and the performance of the CTD.  The points that I used are in the spreadsheet botcomp.xls.  I have done a number of plots from this data.

·	I started looking at the intercomparissons between the two CTD's.  The cast pairs that I used were:

| Station | 1030 Cast | 1031 Cast |
| --- | --- | --- |
| T100 | 25 | 26 |
| E100 | 45 | 44 |
| E400 | 46 | 47 |
| E800 | 49 | 48 |
| MB92 | 70 | 71 |
| E300 | 72 | 73 |

The 1/2 meter averaged data from these pairs of casts is in the spreadsheet ctdcomp.xls.  I have not yet had a good look at the data.

The following notes were copied from a text file containing processing notes. The text file has been moved to the History directory. Joe Linguanti, February 21, 2014

Data compiled by Rick Pearson

The 9207 cruise consists of 56 CTD casts collected in the

spring of 1992 by the NOGAP group.  All casts were taken

on the Ice in the Beaufort Sea region working from

Tuktokaktuk.  Two Sea-Bird SBE-19 CTD's were used:

s/n 1030   (outfitted with an Oxygen Probe)

s/n 1031

The data was originally processed in the 12 Line Header

format by Rick Pearson, using the CTD SHELL II running under

DOS.  In December 1994, Rick Pearson converted the data to

IOS HEADER format for archiving.

The 12 Line to IOS Conversion program did not recognize the

following channels:

Channel: i   =  Oxygen Probe Current

Channel: t   =  Oxygen Probe Temperature

Channel: O   =  Oxygen concentration in ml/l

The data collection methods, and data processing steps are

outlined in the Canadian Data Report of Hydrography and

Ocean Sciences No. 129, "NOGAP B.B, Physical and Chemical

Data Collected in the Beaufort Sea and Mackenzie River

Delta, April-May and September, 1992, and Ice Core Data

Collected in 1991-1992"

The text portion of that report, as written by Rick Pearson,

before it went to Robie Macdonald for editing, is saved in

Word for Windows format in the file: report.doc.  This

report file contains numerous tables and figures that are

not convertable to ASCII. The file report.txt contains just

the text portion of the report file in ASCII format. The

file cstnotes.doc contains a table listing how each CTD cast

fits into the overall mission schedule.

Several casts are missing from this data set.  The missing

casts are a result of instrument problems in the field and

have been omitted because the data is not useable.

The file 9207.rpt contains the output from the IOS SHELL

HEADER SUMMARY program.

May 19, 2004 - Joe Linguanti

----------------------------------------

Converted oxygen units from mmol/m^3 (same as umol/L) to mL/L

with the CALIB program by dividing the original oxygen values by

44.6.