| S/N | Max Pressure (dBars) | Pumped Cond Cell | O2 Probe |
| --- | --- | --- | --- |
| 1030 | 200 | Yes | Yes |
| 1031 | 600 | Yes |  |

Table 1 - Sea-Bird CTD Configurations

| CTD S/N | Constant Added to all Salinities | Standard Deviation of Residuals | Count |
| --- | --- | --- | --- |
| 1030 | 0.0145 | .011 | 20 |
| 1031 | 0.0124 | .007 | 22 |

Table 2 - Salinity Corrections and Standard Deviations

| Date | Cast #'s | Oxygen Offset (ml/l) |
| --- | --- | --- |
| April 14 | 1,2 | .985 |
| April 17 | 3,4,6,7 | -.681 |
| April 19 | 8,9,10,11,12 | -1.791 |
| April 20 | 13,14,15,16,17,18,19,20 | -2.347 |
| April 23 | 21,22 | 1.768 |
| April 24 | 24 | 1.690 |
| April 25 | 25 | 1.612 |
| April 26 | 27,28 | 1.533 |
| April 27 | 29,30 | 1.455 |
| May 3 | 43 | .984 |
| May 4 | 45,46,49 | -1.556 |
| May 5 | 50,51 | -1.373 |
| May 7 | 54,55 | -1.006 |
| May 8 | 62,63,64,65,66 | -0.823 |
| May 9 | 67,68,69 | -0.640 |
| May 11 | 70,72 | -0.273 |
| May 12 | 74 | -2.900 |

Table 3 - Offset Corrections Made to Dissolved Oxygen Data

| Standard Deviation of Residuals (ml/l) | Count |
| --- | --- |
| 0.35 | 120 |

Table 4 - Dissolved Oxygen Standard Deviation

| Station | 1030 Cast | 1031 Cast |
| --- | --- | --- |
| T100 | 25 | 26 |
| E100 | 45 | 44 |
| E400 | 46 | 47 |
| E800 | 49 | 48 |
| MB92 | 70 | 71 |
| E300 | 72 | 73 |

Table 5 - Casts Used for CTD Intercomparisons

| Quantity | Mean | Std. Dev. | Count |
| --- | --- | --- | --- |
| Temp Diff (1030-1031) deg C | -.0071 | .0025 | 132 |
| Sal. Diff (1030-1031) PSU | -.0024 | .0054 | 128 |

Table 6 - Mean and Standard Deviations of the Temperature and Salinity Differences (1030-1031)

| Cast # | Station | CTD S/N | Trans-port * | Bottle on line with CTD? | Chem-istry Done At Stat-ion? | Valid? | Use? | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | E600 | 1030 | T |  |  |  |  | Bad Pump |
| 2 | E800 | 1030 | T |  |  |  |  | Bad Pump |
| 3 | E800 | 1030 | T |  |  |  |  | Bad Pump |
| 4 | E800 | 1030 | T |  |  |  |  | Only one profile |
| 5 | R292 | 1031 | T |  |  |  |  | Bad Pump |
| 6 | R292 | 1030 | T |  |  |  |  | Bad Pressure |
| 7 | R192 | 1030 | T |  |  |  |  | Bad Pressure |
| 8 | E100 | 1030 | H |  |  |  |  |  |
| 9 | E100 | 1030 | H |  |  |  |  | Not a complete profile, for bottle comparison only |
| 10 | E200 | 1030 | H |  |  |  |  |  |
| 11 | E200 | 1030 | H |  |  |  |  | Not a complete profile, for bottle comparison only |
| 12 | E300 | 1030 | H |  |  |  |  | Did not find 18m basin. Cast done in only 5m of water |
| 13 | E400 | 1030 | H |  |  |  |  |  |
| 14 | E400 | 1030 | H |  |  |  |  | Not a complete profile, for bottle comparison only |
| 15 | E500 | 1030 | H |  |  |  |  |  |
| 16 | E600 | 1030 | H |  |  |  |  | Poor salinity intercomparison between profiles |
| 17 | E630 | 1030 | H |  |  |  |  |  |
| 18 | E900 | 1030 | H |  |  |  |  | Bad Pumping |
| 19 | E800 | 1030 | H |  |  |  |  |  |
| 20 | E700 | 1030 | H |  |  |  |  |  |
| 21 | T100 | 1030 | S |  |  |  |  | Bad Pressure |

* T=Twin Otter, H=Helicopter, S=Snow Mobile

Table 7 - CTD Cast Information

| Cast # | Station | CTD S/N | Trans-port * | Bottle on line with CTD? | Chem-istry Done At Stat-ion? | Valid? | Use? | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | T100 | 1030 | S |  |  |  |  | Bad Pressure |
| 23 | T100 | 1031 | S |  |  |  |  | Bad Pressure |
| 24 | T200 | 1030 | S |  |  |  |  |  |
| 25 | T100 | 1030 | S |  |  |  |  |  |
| 26 | T100 | 1031 | S |  |  |  |  |  |
| 27 | ROB0 | 1030 | S |  |  |  |  | Bad Pumping |
| 28 | E300 | 1030 | S |  |  |  |  | Bad Pumping |
| 29 | E900 | 1030 | T |  |  |  |  | Bad Pumping |
| 30 | E900 | 1030 | T |  |  |  |  |  |
| 31 | E230 | 1031 | S |  |  |  |  | Evidence of a lot of turbulence |
| 32 | E220 | 1031 | S |  |  |  |  |  |
| 33 | E110 | 1031 | S |  |  |  |  | Batt failed during profile, did not measure entire water column |
| 34 | E200 | 1031 | S |  |  |  |  | Batt Fail |
| 35 | E200 | 1031 | S |  |  |  |  |  |
| 36 | E210 | 1031 | S |  |  |  |  |  |
| 37 | E660 | 1031 | S |  |  |  |  |  |
| 38 | E650 | 1031 | S |  |  |  |  |  |
| 39 | E640 | 1031 | S |  |  |  |  |  |
| 40 | E630 | 1031 | S |  |  |  |  |  |
| 41 | E620 | 1031 | S |  |  |  |  |  |
| 42 | E600 | 1031 | S |  |  |  |  |  |
| 43 | E500 | 1030 | T |  |  |  |  |  |
| 44 | E100 | 1031 | T |  |  |  |  |  |
| 45 | E100 | 1030 | T |  |  |  |  |  |
| 46 | E400 | 1030 | T |  |  |  |  |  |
| 47 | E400 | 1031 | T |  |  |  |  |  |
| 48 | E800 | 1031 | T |  |  |  |  |  |
| 49 | E800 | 1030 | T |  |  |  |  |  |
| 50 | E670 | 1030 | T |  |  |  |  |  |
| 51 | E200 | 1030 | T |  |  |  |  |  |
| 52 | E665 | 1031 | H |  |  |  |  | No Pump |
| 53 | E663 | 1031 | H |  |  |  |  | No Pump |

* T=Twin Otter, H=Helicopter, S=Snow Mobile

Table 7 (con't) - CTD Cast Information

| Cast # | Station | CTD S/N | Trans-port * | Bottle on line with CTD? | Chem-istry Done At Stat-ion? | Valid? | Use? | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54 | R192 | 1030 | T |  |  |  |  |  |
| 55 | R292 | 1030 | T |  |  |  |  |  |
| 56 | E400 | 1031 | S |  |  |  |  | No Pump |
| 57 | E370 | 1031 | S |  |  |  |  | No Pump |
| 58 | E360 | 1031 | S |  |  |  |  | No Pump |
| 59 | E350 | 1031 | S |  |  |  |  | No Pump |
| 60 | E341 | 1031 | S |  |  |  |  | No Pump |
| 61 | E330 | 1031 | S |  |  |  |  | No Pump |
| 62 | E225 | 1030 | T |  |  |  |  | Second profile used |
| 63 | E300 | 1030 | T |  |  |  |  |  |
| 64 | E661 | 1030 | T |  |  |  |  |  |
| 65 | E662 | 1030 | T |  |  |  |  |  |
| 66 | E670 | 1030 | T |  |  |  |  |  |
| 67 | E300 | 1030 | T |  |  |  |  |  |
| 68 | SITL | 1030 | T |  |  |  |  |  |
| 69 | SITL | 1030 | T |  |  |  |  |  |
| 70 | MB92 | 1030 | T |  |  |  |  |  |
| 71 | MB92 | 1031 | T |  |  |  |  |  |
| 72 | E300 | 1030 | T |  |  |  |  |  |
| 73 | E300 | 1031 | T |  |  |  |  |  |
| 74 | E800 | 1030 | T |  |  |  |  |  |

* T=Twin Otter, H=Helicopter, S=Snow Mobile

Table 7 (con't) - CTD Cast Information

| Station | Cast# | Station | Cast# | Station | Cast# |
| --- | --- | --- | --- | --- | --- |
| E100 | 45 | E400 | 46 | E700 | 20 |
| E110 | 33 | E500 | 15 | E800 | 49 |
| E200 | 51 | E600 | 42 | E900 | 30 |
| E210 | 36 | E620 | 41 | MB92 | 70 |
| E220 | 32 | E630 | 40 | R192 | 54 |
| E225 | 62 | E640 | 39 | R292 | 55 |
| E230 | 31 | E650 | 38 | SITL | 69 |
| E300 | 72 | E660 | 37 | T100 | 25 |
| E330 | 61 | E661 | 64 | T200 | 24 |
| E341 | 60 | E662 | 65 |  |  |
| E350 | 59 | E663 | 53 |  |  |
| E360 | 58 | E665 | 52 |  |  |
| E370 | 57 | E670 | 66 |  |  |

Table 8 - Representative CTD Casts for the Survey Area