PROJECT REPORT

Department of Fisheries and Oceans. 
Science Branch, Pacific Region

To: Regional Science Director			Final:  yes

Monthly:

1. Project No.:  9309		2. Dates:  May 14 to June 1, 1993

3. Project Names:    WOCE Lines PR5 & 6

JGOFS  "New and Regenerated Primary Production in the NE Pacific"

4. Area(s) of Operation:  NE Pacific, along Lines P and R

5. Platform:	John P. Tully		6. Ship's Master:  John Anderson

7. Days Allocated:  21		8. Days of Operation:  20

9. Days lost to weather: none

10. Days lost to other causes: (explain)  ship maintenance, 0.5 d

11. Suitability of platform: only option for deep ocean sampling on Canadian West Coast (station holding capabilities, large A-frame, work deck space for winches, experienced crew, sea water loop)

12. Comments and recommendations: (safety, vessel operation, overtime, etc.)

An excessive number of novice ship's crew (3 deck hands and 2nd mate) created dangers not usually faced.  Our CTD was pulled tight into the block when an inexperienced deck hand did not pay attention (despite a warning 50 m from the surface).  This almost happened a second time with the same person.  The inexperience of the 2nd officer was compensated for by the vigilance of the captain, who was on the bridge frequently to oversee all challenging work.  Still, some routine work was abandoned when lines were drifting under the ship.

Most people working with the new Rosette/winch system want to be convinced that we are working with safe loads.  Loads (both static at depth, and peak in surface waters) appear to exceed safe working limits.  However, there are few options if we are to continue with deep ocean Rosette sampling (heavier wire, new winch drum, new spooling gear would likely be cost prohibitive?).

Suggestions:  Limit the number of novice ship's crew on a cruise, and provide some preliminary instruction on what doing oceanographic sampling entails.  For example, John Campbell could discuss station keeping for hydro casts and CTDs with a new mate.

Confirmation that our Rosette system is typical of those usually used for deep ocean sampling, and guidelines as to what is a safe load on CTD cable need to be supplied to the ship and scientists onboard these cruises.

13. Primary Institute:  Institute of Ocean Sciences (Ocean Physics and Centre for Ocean Climate Chemistry)

14. Associate Institute(s):  University of  B.C., Oceanography

15. Chief Scientist / Affiliation:  FrankWhitney, Climate Chemistry (WOCE)

16. Science Staff / Affiliation: (list names)

| Name | Institute | Responsibility |
| --- | --- | --- |
| Frank Whitney | IOS, COCC | Nutrients, chief scientist |
| Ron Bellegay | IOS, COCC | Sediment trap moorings, sampling |
| Tim Soutar | IOS, COCC | pCO2, electronics, sampling |
| Reg Bigham | IOS, OP | Current meter mooring, sampling |
| Bernard Minkley | IOS, OP | Oxygen, sampling, data |
| Marie Robert | McGill U. | Rosette & CTD sampling |
| Nick Hall-Patch | Contractor | CTD/Rosette electronics & sampling |
| Philip Boyd | UBC | Primary Productivity |
| Diana Varela | UBC | N cycling |
| Hugh MacLean | UBC | sampling |
| Jinping Wu | UBC | N & C transport |
| Sean Doherty | UBC | Nano-plankton |
| Mark Wen | UBC | Zooplankton |
| Steven Ruskey | UBC | Lab assistant |

17. Cruise / Project Objectives: (attach narrative if required).

Complete WOCE Lines PR5 and PR6

Service moorings at P04, P20 and P26

Measure growth rates and activity of  upper ocean biota (JGOFS)

Measure natural fractionation of N and C isotopes along Line P

Measure carbon parameters along Line P

18. Cruise / Project Results: (attach narrative if required).

All programs were in general completely successful.  Equipment failures may reduce the quality of some results.

Marie Robert used her thesis results to check for a temperature abnormalities along Line P, and found surface layer warming of about 1.5o C compared with her mean data for May.  This anomaly is nearly as large as the last El Niño signal.

As IOS' WOCE Hydrography moves towards its major task in 1994, halting progress is being made in developing deep ocean Rosette sampling.  Further tests with the Lantec winch convince us that with minor modifications, this will be a very successful tool.  Spooling appears excellent when wire is under tension.  A different heave compensation system is required urgently.  The present bungie system is a great aid in damping peak loads when the ship pitches (we proved that beyond question by trying a Rosette cast without the bungies in moderate swell, observing a heavy tug on the cable every time the Rosette weighted the line).  However, it needs to be replaced because it requires an extra deck hand be on watch for every deployment and recovery (3 crew required), it is not stiff enough to handle higher loads, and its hardware is a danger on deck.  A system that is mounted on the A-frame seems preferable.

The biologically productive waters we observed in March this year were again evident in May.  Salps and the beautifully blue-sailed Velella occurred in large numbers from the coast, out past station P20.    As usual, nutrients were being upwelled in Juan de Fuca Strait, but were quickly utilized before being transported to coastal waters, so that NO3&NO2 was depleted over most of the shelf (figure below).  Unusually high chlorophyll patches (0.6 to 0.7 mg m-3) were measured in surface samples between stations P22 and 24, and at 30 to 50 m depth at St. P.

Surface NO3&NO2 and Chlorophyll a along Line P

Phytoplankton, nanoflagellates, bacteria and macrozooplankton were all sampled along Line P.  The effects of grazing pressure on growth of various populations was the focus of several onboard experiments.

Using trace metal clean procedures, UBC conducted a number of culture experiments in deck incubators, to assess the importance of several factors on primary production.  Judging by the colour of some of the filters from these studies, something they did produced rapid phytoplankton growth.

Jinping Wu conducted some real chemistry in the lab by refluxing 15NH4 and 15NO3 from sea water and trapping it on zeolite.  His 3 boiling pots and heating mantles taped to the bench entertained all.

19. Equipment:

SAIL:	yes		ADCM:  yes		Other (scientific):  echo sounder

Fishing activity:  none

Winches: 1. (type / ID No.)	Lantec		Wire Type:  CTD

Wire Condition:  heavily used, intended for testing Rosette

Condition of winch at acceptance on vessel:  touch controls, poor break releasing, rust on contacts in control panel, no depth readout.

Repairs/modifications whilst aboard:  Tim Soutar trimmed the controls by reducing their sensitivity (a control panel adjustment) and by balancing the remote control lever so the brake would release before the drum was powered.  These changes made the winch a pleasure to use (compared with the 329 winch).

Recommended repairs/modifications:  rust prevention in control panel, change of wire, repair crack in motor tensioning mount lower left area on back of winch, mount load and wire readouts on winch

Equipment deployed:  Rosette and CTD

No. of casts. 25         Average depth:  3000 m (max 3600 m)

Winches: 2. (type / ID No.)  329, #1482	Wire Type: CTD

Wire Condition:   new

Condition of winch at acceptance on vessel: wire spooling remains a problem.  Using this winch cost us about 1/2 hour extra and included an extra person on each cast to adjust spooling.

Repairs/modifications whilst aboard:  none

Recommended repairs/modifications:  spooling must be made more accurate for deep casts

Equipment deployed:  CTD and Rosette

No. of casts.  25         Average depth:  3000 m (max 3700)

Winches: 3. (type / ID No.)  420 hydro, #1432    Wire Type: hydro

Wire Condition:  break in one of 3 strands at 2500 m

Condition of winch at acceptance on vessel:  working well

Repairs/modifications whilst aboard:  none

Recommended repairs/modifications:  replace wire

Equipment deployed:  nets, Go-Flo samplers

No. of casts. 20	Average depth: 200 m

Winches: 4. (type / ID No.)  329  #1306 	  Wire Type:  none

Wire Condition:

Condition of winch at acceptance on vessel:   good

Repairs/modifications whilst aboard:   none

Recommended repairs/modifications:

Equipment deployed:  bungie system, kevlar mooring lines

Winches: 5. (type/ID) Spooling #1451   Wire Type:  mooring lines

Wire Condition:  user supplied

Condition of winch at acceptance on vessel:  good

Repairs/modifications whilst aboard:   none

Recommended repairs/modifications:  none

Equipment deployed:  5 moorings

Equipment Deployed/Recovered

| Description | Recovery | Deployment |
| --- | --- | --- |
| QEP Mooring | 5 current meters | 5 current meters, P20 |
| 800 m Sed. Trap | none | 1 tap, P04 |
| Free drifting sediment traps | 18 samples intact | 9 bongo traps to 1000 m |
| 3800 m Sed. Trap | 17 samples intact | 1 trap, P26 |
| 200 and 1000 m traps | none | 2 traps, P26 |

Specialized Equipment:

Onboard analysis included salinity (Portasal), oxygen (autotitration), nutrients (4 channel Autoanalyzer), pCO2  (gas chromatograph) and chlorophyll (fluorometry).

All water sampling for WOCE was done with a 24 bottle Rosette and Guildline 8737 CTD to a maximum depth of 3600 m.

A Sea-Cat CTD/Transmissometer from D. Farmer provided information on particle density (plankton) in the upper 150 m at several stations.  Dave Farmer also kinldy lent UBC a 100 kHz Ross sounder to study the vertical migration of zooplankton.

Rod Forbes ( more thanks) installed a flow-through fluorometer, connected to the ship's sea water loop, and with help of  Electronics (Bill Hinds et al), logged data onto SAIL for the first time.

20.  Comments on Equipment

A bungie system is used to dampen peak loads on CTD cables caused by the ship's pitching.  Below about 2000 m, the bungies are fully stretched under static loads.   Because of this, we felt that the bungies likely were unnecessary for Rosette casts, and subsequently removed them before a cast at R09.  However, we were quickly proven wrong.  During the ship's pitching, the Rosette's weight is removed from the cable, then sharply reapplied as the stern of the ship rises on a wave crest.  When the cable was loaded like this, very sharp increases in cable tensions were recorded (2 to 3 times the static load in surface waters, or up to 3500 lb).  A heave compensator mounted on the A-frame seems to be a good option, and must be pursued before the next Line P cruise.

Some problems with the bungie method of heave compensation include the danger of this gear thrashing around on deck, the need for a third deck hand to operate a additional winch used with this system, and the overloading of bungies at depth which reduces its effectiveness.

On one attempted hydro cast to 3800 m, we found a break in one of the 3 wire strands when 2500 m of wire was out.  We taped the break and carefully retrieved our equipment.  I then restricted use of the hydro winch to 500 m, which was adequate for all JGOFS work, but eliminated deep hydro sampling.

The Lantec winch has a crack in a bracket that is used to tension the drive chain.  Also, we were losing data from our precision CTD when using the cable on the Lantec.  Because of these problems, we only used the 329 winch along Line R.

Note: data loss may not be due to this cable, still the cable is old and needs to be retired.

The echo sounder is one piece of equipment we totally depend on, since WOCE requests that CTD profiles reach to within 50 m of the bottom on each cast.  It is disturbing that we can lose such equipment in mid-cruise.

The power supply for the Lantec (I believe we had PGC's unit) needs maintenance soon.  An alarming amount of rust was found when the control panel was opened.  The remote control for the Lantec was desensitized by adjustments made within the panel.  Additional balancing of the remote allowed operators to release the winch brake before applying power to the drum.

A new Intelligent Rosette was tested for NOGAP at Station P.  Due to an electrical short in a connector on the CTD cable, we could not test bottle tripping.  The deck unit for this Rosette was damaged by the short, and is inadequately protected against this type of problem.

Connecting and disconnecting CTDs from the Rosette many times along Line P likely sped up the failure of the connector at the Rosette end of the Lantec cable.  This practice should be rethought.

21.	Comments and Recommendations: (detail any issues not covered above).

I am continually impressed by the skills and abilities of the people who support these sea-going programs.  We faced problems with almost every key piece of equipment during our 18 days at sea.  Winches, wire, CTDs, moorings and lab equipment all had to be modified or repaired on short notice, or we would have lost large portions of our programs.  Tim Soutar made the Lantec winch controls useable early in the trip, by first tweeking the control panel then the winch's remote control.  He then kept us alive for our return leg by repairing the echo sounder when it went haywire at Station P.  Nick Hall-Patch ended up being one of the busier people onboard, with his continual trouble shooting of failing CTD conductivity cells, cables and deck units added onto his basic duties of overseeing Rosette casts and verifying CTD data quality.  Bernard Minkley, when he wasn't sampling, titrating, data crunching, etc., was helping Nick with cable splices and cell maintenance.  Reg pulled out his grab bag to make various bridles for Rosettes that we hadn't had time to prepare on shore.  When we started to run out of parts for rapairs, Hugh MacLean pulled out his sea bag to supply CTD connectors.  The ship's oiler, Al Rosie, helped by repairing a tubing pump (by brazing a drive shaft that had worn!) and keep our NH4 analyses running and also pointed out that our winch was cracking.  The bosun and his regulars worked out routines with us to more safely deploy equipment and spool wire.  Endless discussions about heave compensation, complete with a photo copy of a review of these systems from the captain, produced good directions to pursue in our efforts to reduce load surges.

It is evident from our problems and successes on this cruise, that an electronics person familiar with our CTDs, and an improved kit of spare parts, are essential to the success of our WOCE program.  Our sampling routines are still being developed, and will rely on further testing at sea to become rugged procedures.

22.  Submitted by:	Frank Whitney			  Date:  June 3, 1993

-To be completed by Chief Scientist for each cruise.

-Submit completed WP copy on disc to L. Mcdonald, Dir. Office,IOS -Comments on format of "Project Report" should be submitted

to Chairperson, Science Vessel Users Committee, IOS or to Chairperson, BSB Vessel Users Committee, PBS.