## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 20-Aug-2012 | - Added chemistry files containing Nutrients, Oxygen and NH4 to this cruise. These data had not been previously archived. Frank Whitney entered the data from the analysis and field sheets into a spreadsheet file that is included in the .DOC directory. The spreadsheet file was converted to IOS Header format files. J.L. |

CTD processing notes                                                                          Dec 29th, 1995

Mission: 95-27

Sep 11 - Sep 28, 1995 UTC

Project: Saanich Inlet

119 processed files.

There are 231 original raw files, cast numbers 1-231.

CTD probe 53501 (with transmissometer) was used from cast numbers 1-121

and 157-231.

CTD probe 43825 was used from cast numbers 122-156.

1.  Calibration Coefficients

For casts 1 - 121 and 157 - 231 (CTD probe 53501)

Pressure           = 0.0000000e+00 +  0.2000000e+04 * rawdata

Temperature        =-0.2130000e-02 +  0.1000352e+01 * rawdata

Conductivity_ratio =-0.2830000e-02 +  0.1003384e+01 * rawdata

Transmissivity     =-0.1003910e+03 +  0.2008630e+01 * rawdata

For casts 122 - 156             (CTD probe 43825)

Pressure           = 0.0000000e+00 +  0.1000000e+04 * rawdata

Temperature        = 0.0000000e+00 +  0.1000000e+01 * rawdata

Conductivity_ratio = 0.0000000e+00 +  0.1000000e+01 * rawdata

2.  Surface pressure offset

Each cast needed pressure offset. Offset value for each cast was written

on the header.

3.  DELETE - Swell removal program

- the following parameters were used for all casts:

Program option (3) was chosen:

Surface records removed

Records removed prior to last pressure minimum

Surface pressure max (relative): 0.00

Surface swell ressure tolerance:  1.00

Pressure filtered over width   : 11

Swells deleted, warning message:  1.00

Slow drop rate NOT deleted.

4.  Editing

All files needed some editings. The program CTDEDIT was used.

Cast number    Deleted records/salinity   pressure

----------     ------------------------   --------

5             records                 0 - 5

21                "                    0 - 10

51                "                    0 - 6

55             salinity                0 - 6.5

57                "                    0 - 3

59             records                 0 - 4

63                "                    0 - 13

65                "                    0 - 9.5

67                "                    0 - 3.5

69                "                    0 - 10

71                "                    0 - 6

97             salinity                0 - 14

101             records                 0 - 1

105                "                    0 - 5

103                "                    0 - 12

107                "                    0 - 5

109                "                    0 - 5

113                "                    0 - 8

115                "                    0 - 6

117        No data between 0 27 dbars

118             records                 0 - 6

119                "                    0 - 3

128                "                    0 - 1

130                "                    0 - 12

134                "                    0 - 1

138                "                    0 - 5

140                "                    0 - 1

143                "                    0 - 1

149                "                    0 - 1

151                "                    0 - 7

153                "                    0 - 3

155                "                    0 - 8

159                "                    0 - 4

165                "                    0 - 4

167                "                    0 - 3

171                "                    0 - 2

177                "                    0 - 2

179                "                    0 - 5

185                "                    0 - 2

198            salinity                 0 - 2

200                "                    0 - 2

206            records                  0 - 4

210                "                    0 - 4

224                "                    0 - 4

230            salinity                 0 - 6

5.  Bottle calibrations

0.016 ppt was subtracted from salinity values from cast number 122 to 156.

No temperature correction.

No salinity or temperature correction for cast number 1 to 121 and

157 to 231.

6.  BINAVE

- The following program parameters were used:

Bin channel       : pressure

Averaging interval: 1.0

Minimum bin value : 0.0

Averaged values (not bin values) used

Interpolated values NOT used for empty bins

7.  Particulars

The following were upcasts.

2,4,6,8,10

12,14,16,18,20

22,24,26,28,30

32,34,36,38,40

42,44,46,48,50

52,54,56,58,60

62,64,66,68,70

72,74,76,78,80

82,84,86,88,90

92,94,96,98,100

102,104,106,108,110

112,114,116,

123,125,127,129

131,133,135,137,139

142,144,146,148,150

152,154,156,158,160

162,164,166,168,170

172,174,176,178,180

182,184,186,188,190

193,195,197,199

201,203,205,207,209

211,213,215,217,219

221,223,225,227,229

231

8.  Data processing squences

For cast numbers 1 - 121 and 157 - 230

*.RAW => *.CAL => *.DEL => *.EDT => *.AVG => *.REM => *.CTD => *.STP

(renamed)

For cast numbers 122 - 156

*.RAW => *.CAL => *.DEL => *.EDT => *.AVG => *.REM => *.COR => *.CTD =>*.STP

(renamed)

Andrew Lee

Data archived on tapes V02662 and U02668.

Joe Linguanti