file:971500015.DOC

date: Oct 02,1997

Temperature anomaly at Station P04 (profiles 11, 13,15)

-------------------------------------------------------

re: 960010015

Claude Savenkoff noted a large region (75 metres to 120 metres depth) where

the temperature seemed anomalously high, when compared to previous stations

at this location:

96010011 P04 21/02/96 22:30 UTC WOCE CTD/Rosette

96010013 P04 22/02/96 00:50 UTC WOCE CTD/Rosette

96010015 P04 23/02/96 03:08 UTC Guildline Standard CTD only

I have reviewed the data and can find no reason to EXCLUDE these data.

Although a different CTD was used, the agreement is good in the surface mixed

layer and at depth amongst these casts. Furthermore, a simple error in

temperature measurement should cause a large error in slainity, which

does not seem to be the case. Imspection of the T-S plots does not

reveal unstable water structure.

RMB