## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 10-Jan-2006 | Surface loop data was added to the archive. The data was acquired from John Morris at PBS. The original spreadsheet file from John and more detailed processing notes can be found in the “Cruise_Data\Documents” directory. Any questions regarding this data should be directed to John Morris. J.L. |

CRUISE#: 9640

DATES: 3-24 OCT 1996

_____________________________________________________________________

Cruise particulars: 	High Seas Salmon project

Chief Scientist:		David Welch (PBS Nanaimo)

Vessel:			CFV "W. E. Ricker"

Processing by:		Bon van Hardenberg

Date:				20 January 1997

Number of casts:	 	89

note: preliminary processing done by Robin Brown including generation of

CRUISE SUMMARY, CRUISE TRACKS, CALIBRATION SUMMARY

01/20/97

1. in header files (9640***.hdr):

after inspecting chronology in summary info (hdr_sum.log),

corrected cast numbers 5,17,18,28,41,53 and 77

for CAST#83 corrected longitude to 139 26.14 (was 138 26.14)

corrected LAT,LONG entries for missing hemisphere letters (W,N)

and ran speed check - high values at stns. 12, 39, 40, 77

2/25/97

Manually edited *.CAL files to remove negative values at surface (not removed by DELETE program for nearly all 89 casts). Manually removed 1172 lines from ast#12 which ran partway down then back to surface with bad conductivity values (cover left on?). Ran DESPIKE, DELETE, BOX and BINAVERAGE with values used in processing of 9505.

2/26/97

Cruise name "HS96" in header conflicts with IOSSHELL (expects 4 digit integer) - replaced in *.BOX files with designated IOS cruise number 96-40. Produced preliminary page-plots.

2/27/97

Edited *.BOX files after inspection of stability in TS plots:

Stn. 18: corrected bad conductivity signal (unstable in 45-60m) to uniform over mixed layer (possibly first indication of conductivity cell failure found in following casts 20, 21, 22).

Stns. 20-21-22: removed salinity channel (bad conductivities) from *.AVG files.

Stn. 23: Guildline CTD #53977 replaced by #57743 for this and following casts.

Stns. 23 & 25: Edited surface values based on TS

Stn. 35: Colder water at surface possible close to shore? noted, but not edited.

Stns. 45-48 and 56-57: shallow casts look very noisy, very small dynamic range.

Stn. 73: edited near-surface values (upper 2 m.)

Re-ran BINAVG program for the corrected stations, removed bad salinity and conductivity channels from *.AVG files for casts 20-21-22, renamed from *.REM to *.AVG .

Thinned data to standard depths and produced derived quantities Theta, SVA, Gamma and SoundSpeed for tables. Produced page plots from final bin-averaged data.

Produced a set of T and S contour plots for each of the 15 'lines' identified in the track plots.

note: bottle salinities were collected at surface only, not suitable for comparison with CTD (high variability).