## REVISION NOTICE TABLE

| ### DATE | ### DESCRIPTION OF REVISION |
| --- | --- |
| 10-Jan-2006 | Surface loop data was added to the archive. The data was acquired from John Morris at PBS. The original spreadsheet file from John and more detailed processing notes can be found in the “Cruise_Data\Documents” directory. Any questions regarding this data should be directed to John Morris. J.L. |

# PROCESSING NOTES

Cruise: 9735

Agency: PBS/IOS

Project: RICKER HISEA

Geographic Area: N.E. Pacific, Gulf of Alaska

Chief Scientist: David Welch  (Party Chief:  John Morris)

Platform: W. E. Ricker

Date of Cruise: 2 November 1997 - 8 December 1997

Processed by: Germaine Gatien

Date of Processing :12 January 1998 - 21 January 1997

Number of original casts: 214

Number of casts processed:  107

# INSTRUMENT SUMMARY

One CTD was used for the entire cruise.

Instrument Type:  Guildline 8715 CTD

Instrument Serial Number: 58483

# SUMMARY OF QUALITY AND CONCERNS

There were problems with the thermosalinograph data because air locks occurred in the sea water pump in  seas over 2-3 metres. This affects the surface salinity calibrations.

Bottle calibrations consisted of only 2 deep bottles and 4 in the mixed layer. It appears that the CTD read high by about 0.015 but there is insufficient data to justify recalibration.

The value of deep-water transmissivity varies from 65% to 70%, with the exception of cast #166 where it is was only 55%.  After cast #24 the transmissometer was thoroughly cleaned, and note is made in the log that light transmission then increased by 4%. The values did remain higher until cast #49, but after that the peak value varies considerably.

As has been seen in previous cruises the deep transmissivity tends to decrease with depth on down-casts  by about 0.025% per 100m. It appears to decrease with decreasing depth on the up-casts though this has not been studied closely because these casts have not been processed. Plots are included of casts 69,70,71 and 72 to illustrate.  These were chosen as casts where there is little structure at depth. For many casts there is considerable structure in transmissivity at depth, especially around 600m. This may be related to overwintering of zooplankton.

# PROCESSING SUMMARY

- PRELIMINARY STEPS

The Log Book was obtained and  note was made of problems that occurred during the cruise.

The salinity bottle data sheets were found.

The cruise summary sheet was completed.

The header summary and header check were examined. (Before this stage a comment in the cast #1 header section was removed; it incorrectly stated that there was no transmissivity data for that cast.) An error in identifying the Instrument type was fixed in casts 1, 3, 5 and 7.

The surface check gave an average surface pressure of 6.5db.

- CALIBRATION

See the attached sheet for a list of the calibration constants used. These include a post-cruise transmissivity calibration.

The pressure was offset by -6.5db , the value indicated by program SURFACE. After this step the average surface pressure was 0.090db.

- DESPIKE

- The following DESPIKE parameters were used:

- CHANNEL        FIT   OVER-  MIN       MAX       MIN         MAX         SPIKE

- WIDTH LAP    VALUE   VALUE  STDDEV  STDDEV     TOL.

- -------------------- ----- ----- ------- ------- ------- ------- -----

- Pres 		55     5        -2.00       4000.00	.10000        999.000    3.00

- Temp   	55     5        -2.00          15.00	.00500        999.000    3.00

- Cond    	55     5           .00            1.50	.00010        999.000    3.00

- Tran     	55     5           .00        100.00 	.10000        999.000    3.00

- TIME COMPENSATION

- Temp. probe Dist (m):   .0700  Sample period (sec):   .0400

- DELETE

The following DELETE parameters were used:

Surface Record Removal: Low Sal & Last Press Min

Maximum Surface Pressure (relative):      20.00

Minimum Surface Salinity:     10.000

Surface Swell Pressure Tolerance:       1.00

Pressure filtered over width:   11

Swells deleted. Warning message if pressure difference of      2.00

Drop rates<    .30m/s (calculated over  11 points) will be deleted

Sample interval =       .04 seconds.

For casts #5,23,49,51,53,59,61,63,67,77,166,200 and 206 there were warnings of pressure differences of greater than 2; these were not removed by lowering the drop rate to 0.2m/sec so they were run again with the DELETE LOW DROP RATE feature turned off.

- FILTER

- A median box car filter with filter width 21 was applied to the following channels:

- Tr and  sal

- BIN AVERAGE

The following Bin Average values were used:

Bin channel = pressure

Averaging interval =      1.000

Minimum bin value =       .000

Average value will be used

Interpolated values are NOT used for empty bins

8.	THIN

The following Thin parameters were used:

Reference Channel: Pressure

Using Standard Depths for Thin Values

Tolerance for finding values:      1.500

Interpolated Value between Closest Points will be Output

- HAND EDITING AND TEST PLOTS

The following casts were edited for T and S only in the top 10m:  69,91,143,157,159,168,182,204.

The following casts were edited more extensively: 11,15,17,31,37,39,43,49,50,51,57,61,71,73,75,81,89,93,99,

107,111,113,125,137,141,155,161,163,166,170,172,186,192,196,208,210,212,214.

Details are noted in the comment section of the individual files.

- COMPARISON WITH BOTTLE DATA

Only six bottle calibrations were done for this cruise and the results are inconclusive. Four were in the top 50 m and indicate that the CTD was reading high by about 0.011 to 0.017 . Two bottles at 1000m show the CTD high by 0.019 and 0.0369. The loop salinities were compared with surface values at 7 sites where the surface was well-mixed and the records not heavily edited; the CTD values were higher by an average of 0.015 (range: 0.0135 - 0.0166). These values suggest that the CTD read high by about 0.015 in the mixed layer; while it was likely high in deeper water it is impossible to say by how much. Given these uncertainties no recalibration was done.

- OTHER INTERCOMPARISONS

Adjacent casts were examined for variations in S. For casts #19 - #25 the salinity varied by less than 0.03 at 900m over a half a degree of longitude. Casts #63 and #65, which were separated by about 1 degree of longitude, had salinities within 0.03 at 900m. This suggests that the CTD was internally consistent.

- FINAL PLOTS

Page plots were prepared using the edited data.

13.	REMOVE CHANNELS

The following channels were removed from all casts: Conductivity_Ratio.

14.	PRODUCE FINAL FILES

a.) The final files were renamed *.ctd.

b.) A cross-reference listing was produced.

c.) An error in the instrument section of the headers was fixed using HEADEDIT. The instrument model was incorrectly identified as WOCE. In fact, it was model 8715.

15.	PARTICULARS

The instrument type and model are entered incorrectly in the headers for Casts 1-8 and both the instrument model is wrong for casts 9 - 217.

Cast #1		- A comment was deleted from the header  because it mistakenly indicated that there was

no transmissivity data for this cast.

Casts #1 & 2	- The transmissivity calibration for casts 1 and 2 are wrong in the raw files.

Cast 1-24 	- The transmissometer was cleaned before cast 25 and a 2% gain in light transmission was

noted in the log. The earlier casts have a maximum Tr at depth of about 65%. This rises to 		about 70% for the next 8 casts and then varies considerably.

Cast #29	 - An entry of a single letter (e) in the comment section was assumed to be accidental and

was removed.

Casts 49/50	- The salinity was very noisy so the downcast was deleted.; the upcast #50 was used.

Cast 55/56	-Computer crashed; no data file.

Cast 64		-Up and down casts in one file

Casts 141/142	-Large biomass at 100-150m. Showed in Tr and 38Kz sounder.

Cast 163	-Rough sea

Cast 170/171	-Conductivity down trace unstable; uptrace o.k. Delete #170; use 171.

Cast 174/175	-Computer crash at 120m on down cast. Upcast o.k. Delete #174; use 175.

Cast 176/177	-Deep cast to straighten wire on winch spool.

Data archived on tapes DLT046 and DLT047 in saveset 9735_ALL.BAK

Institute of Ocean Sciences

CRUISE SUMMARY

Cruise ID#:	9735

Dates: 	Start:	November 2, 1997			    End:	December 8, 1997

Location:	N.E. Pacific,Gulf of Alaska

Vessel:		W. E. Ricker

Chief Scientist:	David Welch   (Party Chief: John Morris)

| CTD# | Make | Model | Serial# | Used with Rosette? | CTD Calibration Sheet Competed? |
| --- | --- | --- | --- | --- | --- |
| 1 | Guildline | 8715 | 58483 | no | yes |

NOTE: Record Sensor Calibration using the form “CTD Calibration Information”, and attach.

Notes:

Institute of Ocean Sciences

CTD Calibration Information

Make/Model/Serial#:	Guildline /8715/58483

Cruise ID#:	9735

| Calibration Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sensor |  | Pre-Cruise |  | Post Cruise |  |
| Name | S/N | Date | Location | Date | Location |
| Pressure |  |  |  |  |  |
| Temperature | 57915 | July 97 | J L TECH SERVICES |  |  |
| Conductivity | 58814 | July 97 | J L TECH SERVICES |  |  |
| Transmissometer | 254 *** | 2 Aug 97 | lab - BGM | 11 Jan 98 | lab -John Love # |

| Calibration Coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Formula | Coefficients |  |  |  |  |  |
| Name | # | C1 | C2 | C3 | C4 | C5 | C6 |
| Temperature | 10 | .03798 | .9991 |  |  |  |  |
| Conductivity | 10 | .000435 | .99957 |  |  |  |  |
| Transmissivity # | 10 | 101.987 | -2.0408 |  |  |  |  |

Sensor Calibration Notes:

*** - The log incorrectly states that the transmissometer was # 234.

# - The post-cruise transmissivity calibrations were used.